ANSI Standard (X3.64) Control Sequences

for Video Terminals and Peripherals

with Examples for the IBM PC

Compiled by W.C. Parke, 1986

References:

ANSI X3.64-1979:

Information Systems-Codes, American National Standards

Institute, 1430 Broadway, NY, NY 10018

ISO 6429 standard:

Information Processing-ISO 7-bit and 8-bit Coded

Character Sets-Additional Control Functions for

Charater-Imaging Devices, ANSI as above.

Mark L. Siegel, April 1984 BYTE, p. 365

Zenith Programmer's Utility Pack, Ver. 2, 1984

In the table below:

F      is a Final character in

an Escape sequence (F from 48 to 126 in the ASCII table)

a control sequence (F from 64 to 126).

Fe     is a Final character in ASCII range 64 to 95.

Gs     is a graphic character appearing in strings (Gs ranges from

32 to 126) in the ASCII table.

C0     is the set of 7-bit ASCII control characters from 0 to 31.

C1     the set of control characters available only in 8-bit systems.

Ce     is a control represented as a single bit combination in the C1 set

of controls in an 8-bit character set.

I      is an Intermediate character from 32 to 47 (inclusive) in the

ASCII table.

P      is a parameter character from 48 to 63 (inclusive) in the ASCII

table.

Pn     is a numeric parameter in a control sequence, a string of zero or

more characters ranging from 48 to 57 in the ASCII table.

Numeric parameters must be in decimal radix.

Ps     is a variable number of selective parameters in a control sequence

with each selective parameter separated from the other by the code

59 (which usually represents a semicolon); Ps ranges from

48 to 57 and 59.

Default

Sequence     Sequence                                      Parameter

Mnemonic     Name              Sequence                    Value

---------------------------------------------------------------------------

Introducer sequences:

CSI  Control Sequence Intro    Esc [

SS2  Single Shift 2 (G2 set)   Esc N

SS3  Single Shift 3 (G3 set)   Esc O

Delimiters:

APC  Applicatn Program Command Esc Fe

DCS  Device Control String     Esc P

OSC  Operating System Command  Esc ]

PM   Privacy Message           Esc ^

ST   String Terminator         Esc \

Configuration:

DA   Device Attributes         Esc [ Pn c                   0

The VT100 responds to receiving the DA by transmitting the

sequence:  Esc [ ? l ; Ps c

DSR  Device Status Report      Esc [ Ps n                   0

The console driver will output a CPR sequence.

STS  Set Transmit State        Esc S

DMI  Disable Manual Input      Esc \

EMI  Enable Manual Input       Esc b

INT  Interrupt                 Esc a

RIS  Reset to Initial State    Esc c

Editor Functions:

CBT  Cursor Backward Tab       Esc [ Pn Z                   1

CCH  Cancel Previous Character Esc T

CHA  Cursor Horzntal Absolute  Esc [ Pn G                   1

CHT  Cursor Horizontal Tab     Esc [ Pn I                   1

CNL  Cursor Next Line          Esc [ Pn E                   1

CPL  Cursor Preceding Line     Esc [ Pn F                   1

CPR  Cursor Position Report    Esc [ Pn ; Pn R              1, 1

Report of cursor current line and column.  ()Do not use as part

of MS-DOS Prompt command.)

CTC  Cursor Tab Control        Esc [ Ps W                   0

CUB  Cursor Backward           Esc [ Pn D                   1

Move cursor back Pn columns.  Ignored if cursor in leftmost

column.

CUD  Cursor Down               Esc [ Pn B                   1

Move cursor down Pn lines.  Ignored if cursor on bottom line.

CUF  Cursor Forward            Esc [ Pn C                   1

Move cursor Pn columns forward.  Ignored if cursor at rightmost

column.

CUP  Cursor Position           Esc [ Pn ; Pn H              1, 1

Move cursor to position specified.  First parameter is line

number and second is column number.  No parameters means home

position.

CUU  Cursor Up                 Esc [ Pn A                   1

Move cursur up Pn lines.  Ignored if cursor on top line.

Default

Sequence     Sequence                                      Parameter

Mnemonic     Name              Sequence                    Value

---------------------------------------------------------------------------

CVT  Cursor Vertical Tab       Esc [ Pn Y

DAQ  Define Area Qualification Esc [ Ps o                   0

DCH  Delete Character          Esc [ Pn P                   1

DL   Delete Line               Esc [ Pn M                   1

EA   Erase in Area             Esc [ Ps O                   0

ECH  Erase Character           Esc [ Pn X                   1

ED   Erase in Display          Esc [ Ps J                   0

Erases whole screen and cursor goes to home position.

EF   Erase in Field            Esc [ Ps N                   0

EL   Erase in Line             Esc [ Ps K                   0

Erases everything between the cursor and the end of the line,

including cursor position.

EPA  End of Protected Area     Esc W

ESA  End of Selected Area      Esc G

ICH  Insert Character          Esc [ Pn @                   1

IL   Insert Line               Esc [ Pn L                   1

NP   Next Page                 Esc [ Pn U                   1

PP   Preceding Page            Esc [ Pn V                   1

SD   Scroll Down               Esc [ Pn T                   1

SL   Scroll Left               Esc [ Pn Space @             1

SPI  Spacing Increment         Esc [ Pn ; Pn Space G        none

SPA  Start of Protected Area   Esc V

SR   Scroll Right              Esc [ Pn Space A             1

SSA  Start of Selected Area    Esc F

SU  Scroll  Up                 Esc [ Pn S                   1

TBC  Tab Clear                 Esc [ Ps g                   0

TSS  Thin Space Specification  Esc [ Pn Space E             none

VPA  Vert Position Absolute    Esc [ Pn d                   1

VPR  Vert Position Relative    Esc [ Pn e                   1

VTS  Vertical Tabulation Set   Esc J

Format Sequences:

FNT  Font Selection            Esc [ Pn ; Pn Space D        0, 0

GSM  Graphic Size Modify       Esc [ Pn ; Pn Space B        100, 100

GSS  Graphic Size Selection    Esc [ Pn Space C             none

HPA  Horz Position Absolute    Esc [ Pn `                   1

HPR  Horz Position Relative    Esc [ Pn a                   1

HTJ  Horz Tab w/Justification  Esc I

HTS  Horizontal Tab Set        Esc H

HVP  Horz & Vertical Position  Esc [ Pn ; Pn f              1, 1

Moves cursor to line number, column number specified.

No parameters moves cursor to home position.

IND  Index                     Esc D

JFY  Justify                   Esc [ Ps ; ... ; Ps Space F  0

MC   Media Copy                Esc [ Ps i                   0

MW   Message Waiting           Esc U

NEL  Next Line                 Esc E

PLD  Partial Line Down         Esc K

PLU  Partial Line Up           Esc L

PU1  Private Use 1             Esc Q

PU2  Private Use 2             Esc R

QUAD Typographic Quadding      Esc [ Ps Space H             0

RCP  Restore Cursor Position   ESC [ u

Restores cursor position saved by SCP.

REP  Repeat Char or Control    Esc [ Pn b                   1

RI   Reverse Index             Esc M

Default

Sequence     Sequence                                      Parameter

Mnemonic     Name              Sequence                    Value

---------------------------------------------------------------------------

RM   Reset Mode                Esc [ Ps l                   none

Reset screen width or type.  See SM for parameters.  Parameter 7

will reset wrap mode (characters past end-of-line are lost.)

SCP  Save Cursor Position      Esc [ s

Cursor position will be restored with RCP sequence.

SEM  Select Edit Extent Mode   Esc [ Ps Q                   0

SGR  Select Graphic Rendition  Esc [ Ps m                   0

Invokes graphic rendition specified by the parameters.  All

following characters are rendered according to the parameter(s)

until the next SGR.  Parameter functions on IBM PC:

0  All attributes off (normal, white on black)

1  Bold on (high intensity)

4  Underscore On (monochrome displays only)

5  Blink on

7  Reverse Video on

8  Cancelled on  (invisible)

30 Black foreground

31 Red foreground

32 Green foreground

33 Yellow foreground

34 Blue foreground

35 Magenta foreground

36 Cyan foreground

37 White foreground

40 Black foreground

41 Red background

42 Green background

43 Yellow background

44 Blue background

45 Magenta background

46 Cyan background

47 White background

SM   Select Mode               Esc [ Ps h                   none

Invokes the screen width or type specified.  IBM PC paramters:

0  40 x 25 monochrome

1  40 x 25 color

2  80 x 25 monochrome

3  80 x 25 color

4  320 x 200 color

5  320 x 200 monochrome

6  640 x 200 monochrome

7  wrap at end of line (word wrap)

IBM PC Keyboard Reassignment:

Keyboard reassignment uses an undefined ANSI X3.64-1979 code sequence

to define a character or set of characters send by the console whenever

a given key is pressed on an IBM PC compatible.  The control sequence

is:

ESC [ Pn ; Pn ; ... Pn p

or

ESC [ "string" ; p

The first ASCII code in the sequence defines which code is being mapped.

If the first code is 0 (nul), then the second code is the extended

IBM keycode.  The remaining numbers define the sequence of ASCII codes

generated when this key is intercepted, or the string (enclosed in

quotes) to be generated.  For example:

ESC [65;81p             A becomes Q

ESC [0;68;"dir";13p     F10 generates  dir  followed by a CR.