Data Structures Used in IBM PC Compatibles and the PS/2

Compiled and Annotated by

(c) 1985,'86,'87,'88

Send comments or corrections to:

William C. Parke

1820 S Street NW

Washington, D.C. 20009

----------------------------------------------------------------------------

The following data structures are found in RAM, ROM, on the boot sector

of formated disks, in file headers, or are created dynamically by the

operating system.  The structures establish the fields and parameters

needed by the operating system to initialize and maintain communication

with the various devices or files attached to the system.

_____________________________________________________________________________

-----------------------------------------------------------------------------

PC Data Format:

-----------------------------------------------------------------------------

Bit    = One or zero value.  A bit can be saved as an electrical circuit

latch, through electrical capacitance, by an induced magnetic field,

mechanical change, etc.  All information can be stored as a series

of bits. Use DBIT for asm directive.

Common Synonyms to bit values:

1 = One  Set   High True  Yes On  Full Mark  Open  Pass  Start Active

0 = Zero Reset Low  False No  Off Null Space Close Block Stop  Inactive

Nibble = Four bits, labeled 3 to 0; Half a byte; Used to store a

numerical value from 0 to 9 in Binary Coded Decimal; Used

to store hex value 0-F. (Also called nybble)

BCD    = Binary coded decimal value stored in one nibble as a value between

0 and 9.  The 80x86 has several instructions for transforming

between binary and BCD.

Tribble = Three nibbles; Used in MS-DOS File Allocation Table when the

disk partition has less than 16 Megabytes.

Byte   = Eight bits, labeled 7 to 0.  Each ASCII character has a unique

byte value. The IBM extended character set and the IBM EBCDIC

characters have a unique byte value. The 80x86 registers AH, AL,

BH, BL, CH, CL, DH, DL each hold one byte.  80x86 memory is

accessible at a byte level. Use DB for asm directive.

ASCII Byte = American-Standard-Code-for-Information-Interchange byte.

ASCII codes represent numbers, letters, symbols and controls

using a 7-bit code.

EBCDIC = Extended binary coded decimal information codes established

and used within IBM mainframe computers to represent numbers,

letters, symbols and controls using an 8-bit code.

Packed Decimal = Bytes holding a decimal value (0-9) in each nibble.

Word   = Two bytes, sixteen bits, labeled 15 to 0.  The 80x86 registers

AX, BX, CX, DX, SI, DI, BP, ES, DS, SS, and SP hold one word.

The 80x86 memory is accessible at a word level.  An 80x86 word

is stored low byte first.  Integer words use bit 15 for the

sign.  A 'word' on some computers refers to a value which can

be stored in an arithemetic register of the Central Processing

Unit (CPU). Thus, the bit size of a word varies among computers.

Use DW for asm directive.

Double Word = Two words.  The 80x86 memory can be addressed by a double

word.  The 80x86 can load a double word from memory by a single

instruction.  Double words are stored low word first.  Integer

double words use bit 31 for the sign.  The 8087 stores the

two's complement if the number is negative (i.e. 1 + NOT x).

In this way, a number plus its negative becomes zero. Use DD

for asm directive.

80386 Pointer = 48 bits.  Largest address pointer in 80386.  Use DP for

asm directive.

Quad Word = Four words. Allocation for a long integer. Use DQ for asm

directive.

Ten byte = Allocation for packed decimal.  Use DT for asm directive.

8087 Long Integer = 8 bytes, with last bit labeled 63 carrying the sign:

If the number is positive, bit 63 is 0.  If negative, the

two's complement of the number is stored, making bit 63 a 1.

8087 Packed Decimal = 10 bytes; bit 79 is sign, 78-72 unused, 71-0 are

18 consecutive 4-bit packed decimal values.

8087 Short Real = 4 bytes; bit 31-23 is biased exponent (exponent + 127);

22-0 holds normalized significand (shifted to left until 1

appears at bit 23).  Bit 23 of number is implicit = 1.

8087 Long Real  = 8 bytes; bit 63 is sign of number; 62-52 is biased

exponent (exponent + 1023); bits 51-0 are normalized significand

(bit 52 of number is implicit = 1).

8087 Temporary Real = 10 bytes; bit 79 = sign; 78-64 = biased exponent

(exponent + 16383); bit 63-0 = normalized significand, with

explicit 1 at bit 63.

MASM Long Real DQ = 8 bytes; bits 63-56 = biased exponent (exponent + 129);

bit 55 = sign; bits 54-0 = normalized significand with implicit

1 as bit 55.

MASM Short Real DD = 4 bytes; bits 31-24 = biased exponent (exponent + 129);

bit 23 = sign; bits 22-0 = normalized significand with implicit

1 as bit 23.

Paragraph = 16 bytes; Change of address if an 80x86 segment register

is incremented.

Page = 256 bytes; Change of address if an 80x86 register high byte

is incremented.

Sector = 512 bytes; Smallest number of bytes which can be loaded by

MS-DOS from a disk.

Cluster = A group of sectors which are indexed by MS-DOS as allocation

units; these are reserved for file storage and data on disks.

Allocation Unit = Cluster which has been assigned in a File Allocation

Table (FAT) for a disk.

Kilobyte = 1024 bytes (2**10); Smallest number of bytes in one

allocation unit for a file under MS-DOS.

Segment = 65,536 bytes (2**16); 80x86 address range for fixed segment

register; this range reached by an offset stored in a word

register; The largest address space for an 8080 CPU.

Megabyte = 1,048,576 bytes (2**20); The largest address space for an

8086 CPU; An 80286 can address 16 Megabytes of physical memory;

An average book stores about 1 Megabyte of data in 250 pages.

Gigabyte = 1,073,741,824 bytes (2**30); An 80386 can address 4 Gigabytes

(2**32) of physical memory; A laser disk can hold close to

1 Gigabyte of data; 1000 books of information.

Terabyte = 1,099,511,627,520 bytes (2**40); The virtual address space of

an 80386 CPU is 64 terabytes (2**46); Storage in a large library

is under a terabyte; Estimated storage capacity of the human brain

is about 10 terabytes.

_____________________________________________________________________________

-----------------------------------------------------------------------------

Disk Drive Structures:

-----------------------------------------------------------------------------

Diskette Control Table:

-----------------------------------------------------------------------------

The diskette control table address is stored in the Int 1Eh vector.  It

is used to establish diskette controller parameters during formating,

reading, and writing to a disk.  See Int 13h, function 18h.

Table Definitions:

Offset  Data type Description

00H    BYTE      Disk Controller Mode byte 1:

ssssuuuuB     s=0-FH  = head step rate in milliseconds

u=0-FH  = head unload time in milliseconds

01H    BYTE      Disk Controller Mode byte 2:

llllllldB  l=0-7FH = head load time/2-1 in milliseconds

d=0-1   = DMA flag (0 means DMA is used)

02H    BYTE      Clock Ticks until motor off

03H    BYTE      FM or MFM Mode:  Log2 (Bytes per Sector/128)

FM= Freq. Modulation used on single-density disks

MFM=Modified Freq. Modulation used on double-sided disks

04H    BYTE      Last Sector on Track (e.g. 09H)

05H    BYTE      Gap Length in Bytes

MFM   Bytes/Sector   Sectors/track  Write Gap  Format Gap

1           256             12H      0AH          0CH

1           256             10H      20H          32H

2           512             08H      2AH          50H

2           512             09H      1BH          6CH

3          1024             04H      80H          F0H

4          2048             02H      C8H          FFH

5          4096             01H      C8H          FFH

06H    BYTE      Disk Data Length (80H for 128 bytes/sec or 0FFH otherwise)

07H    BYTE      Gap Length when formatting

08H    BYTE      Data pattern used during formatting (fill byte)

09H    BYTE      Floppy Head Bounce Delay (in milliseconds)

0AH    BYTE      Floppy Motor Start Delay (in 0.125 second intervals)

0BH    BYTE      Floppy Motor Stop Delay (in 0.25 seconds intervals)

----------------------------------------------------------------------------

----------------------------------------------------------------------------

Fixed Disk Drive Parameter Tables:

----------------------------------------------------------------------------

The address for these tables are stored in the Int 41h vector address

for drive 1, and Int 46h vector address for drive 2.  These tables can

be initialized to new drive type characteristics using Int 13h, function

9h.

Offset Data type  Description

00H    WORD      number of tracks (called cylinders on fixed disks)

02H    BYTE      number of heads

03H    WORD      start cylinder for reduced write current

05H    WORD      start cylinder for write precompensation on inner tracks

07H    BYTE      maximum number of bits to correct data errors

(ECC burst length; ECC=Error correcting code)

08H    BYTE      controller options 7 disable access retries

6 disable ECC retries

5-3 000 (non-XT:

5=defect map at max.cyl.+1

3=more than 8 heads on AT)

2-0 000   3 usec/step

100 200 usec/step

101  60 usec/step (approx.)

110   3 msec/step

111   3 msec/step

09H     BYTE      time out value for read/write

0AH     BYTE      time out value for format

0BH     BYTE      time out for disk diagnostics

0CH     WORD      landing zone

0EH     BYTE      sectors/track

0FH     BYTE      reserved

_____________________________________________________________________________

-----------------------------------------------------------------------------

BIOS System Structures:

-----------------------------------------------------------------------------

Hard Disk Primitive Boot Sector:

-----------------------------------------------------------------------------

Rather than a DOS Boot Sector, a hard disk may contain a primitive boot

sector which is loaded into memory by the system ROM as the first disk

activity.  This sector must be the first sector on the disk (cylinder 0,

head 0, sector 1).  It is loaded at 0:7C00H and then run, starting at

0:7C00H. The code and data from this sector is strongly machine

dependent. For IBM machines, the code checks the first byte of the four

16 byte disk drive partition tables starting at 01BEH to find the

designated active and bootable partition.  If one is not found, an Int

18h is generated to enter ROM BASIC.  Otherwise, the active DOS boot

sector is loaded and run.

Offset Data type  Description

000H   n BYTES    Primitive Boot Code to find active DOS partition

1BEH   16 BYTES   First disk drive partition table

...

1FEH   WORD       AA55H = boot record signature

-----------------------------------------------------------------------------

Extended Fixed Disk Drive Parameter Table:

-----------------------------------------------------------------------------

For PS/2 (except Model 30), the fixed disk sector 2 at Head 0, Track 0

contains the information needed to support DOS extended partitions, from

logical drive D: to F:.  The format is:

Offset Data type  Description

000H    1 WORD    Length of Extended Fixed Disk Drive Parameter Table below

002H    22 BYTES  (ID) ASCII string: 'IBM HARDFILE TYPE xxx '

018H    1 BYTE    Drive Type number in binary

019H    16 BYTES  Fixed disk drive partition table

-----------------------------------------------------------------------------

Disk Drive Partition Table:

-----------------------------------------------------------------------------

The disk drive partition tables begin at offset 1BEH in the boot record

of the hard disk (track 0, cylinder 0, sector 1), at offset 019H in the

extended fixed disk drive partition table, and offset 1BEH in the DOS

boot sector.

Offset  Data type Description

00H    BYTE      Partition status: 00=inactive; 80H=bootable and active

01H    BYTE      Starting head of boot code

02H    WORD      Starting sector and cylinder of boot code

(cyl. = bits 15,14,7-0; sector = bits 13-8)

04H    BYTE      Partition type: 1 = DOS with 12 bit FAT

4 = DOS with 16 bit FAT

5 = extended DOS

6 = reserved

0DBH= concurrent DOS

05H    BYTE      Ending head

06H    WORD      Ending sector and cylinder

08H    DWORD     Starting absolute sector for partition

0CH    DWORD     Number of sectors

The 16 byte block repeats for each of four partitions, until:

40H    WORD      AA55H = valid boot record signature (at 1FEH)

which completes the first sector on the disk.

----------------------------------------------------------------------------

DOS Boot Sector:

-----------------------------------------------------------------------------

Offset  Data type Description

00H    3 BYTES   jump instruction to boot code in sector: JMP 0BH + nn

(near JMP for DOS 2.1 (E9H), short JMP for DOS 3.x (EBH) )

03H    8 BYTES   Original Equipment Manufacturer (OEM) name and version

0BH    nn BYTES  Bios Parameter Block as below

bb BYTES  Boot Code

1BEH    64 BYTES  Possible Partition Tables

1FEH    WORD      AA55H boot signature

200H    ...

----------------------------------------------------------------------------

Disk BIOS Parameter Block:

-----------------------------------------------------------------------------

The BIOS Parameter Block (BPB) determines the nature of a disk medium.

It can be found in the DOS boot sector of every formated disk.  This DOS

boot sector is at track 0, head 0, sector 1, on diskettes, and follows

the primitive boot sector and extended disk drive partition table on

hard drives.

A pointer to the memory image of the BPB is passed to a block device

when requesting initialization of the device.  A drive parameter block

can be built with device driver request using the BPB and with the

undocumented function 53h, Int 21h.  See Disk Parameter Block.

BIOS Parameter Block Structure:

Offset  Data type Description

00H    WORD      Sector size in bytes (not less than 32)

02H    BYTE      Sectors/Clusters [Allocation Unit](power of 2)

03H    WORD      Number of reserved sectors (starting at logical sector 0)

05H    BYTE      Number of FATs (usually 2)

06H    WORD      Number of directory entries (maximum allowed)

08H    WORD      Total number of sectors in logical image, including boot

0AH    BYTE      Media descriptor 11111xxxB (ID byte)

0BH    WORD      Number of sectors per FAT

0DH    WORD      Sectors/track

0FH    WORD      Number of Heads

11H    DWORD     Number of Hidden Sectors (used in partitioning schemes)

15H    DWORD     Number of logical sectors if +08H = 0  (DOS 4.0)

DOS 4.0 Boot Sector Extension (created by FORMAT and LABEL commands):

19H    BYTE      Physical drive number

1AH    BYTE      Reserved

1BH    BYTE      Segment byte 29h

1CH    DWORD     Volume serial number (computed from date/time)

20H    11 BYTES  Volume Label

2BH     8 BYTES  Reserved

Example BIOS parameters for drives:

Diskette:

Bytes/  Sec/   Track/ Sides  Bytes  Sec/   Sec/  Sec/  ID

Sector  Track   Side                Cluster DIR  FAT  Byte

.-------+------+------+-----+-------+------+-----+----+----.

8.00 inch disk |  128  |  26  |  77  |  1  | 250K  |  4   | 17  |  6 | FE |

|  128  |  26  |  77  |  1  | 250K  |  4   | 26  |  6 | FD |

| 1024  |   8  |  77  |  2  | 1.23M |  1   |  6  |  1 | FE |

5.25-inch disk |  512  |   8  |  40  |  1  | 160K  |  1   |  4  |  1 | FE |

|  512  |   8  |  40  |  2  | 320K  |  2   |  7  |  1 | FF |

|  512  |   9  |  40  |  1  | 180K  |  1   |  4  |  2 | FC |

|  512  |   9  |  40  |  2  | 360K  |  2   |  7  |  2 | FD |

|  512  |  15  |  80  |  2  | 1.20M |  1   | 14  |  7 | F9 |

3.50-inch disk |  512  |   9  |  80  |  2  | 720K  |  2   |  7  |  3 | F9 |

|  512  |  18  |  80  |  2  | 1.44M |  1   | 14  |  9 | F0 |

`-------+------+------+-----+-------+------+-----+----+----'

Hard Disk:

Bytes/  Sec/   Cylin. Heads  Bytes  Sec/   Sec/  Sec/  ID

Sector  Cylin.                      Cluster DIR  FAT  Byte

.-------+------+------+-----+-------+------+-----+----+----.

XT 10 Mb 5.25" |  512  |  17  |  306 |  4  | 10Mb  |   8  |  32 |  8 | F8 |

AT 20 Mb 5.25" |  512  |  17  |  615 |  4  | 21Mb  |   4  |  32 | 40 | F8 |

PS/2 20 Mb 3.5"|  512  |      |      |     | 21Mb  |      |     |    | F8 |

PS/2 44 Mb 3.5"|  512  |  17  | 1022 |  5  | 44Mb  |  17  |  32 | 43 | F8 |

PS/2 70 Mb 3.5"|  512  |      |      |     | 70Mb  |      |     |    | F8 |

`-------+------+------+-----+-------+------+-----+----+----'

-----------------------------------------------------------------------------

System ROM Parameter Table:

-----------------------------------------------------------------------------

Function 0C0h under Int 15h returns a pointer to this table in ES:BX.

Offset Data type  Description

00H   WORD       byte_count  number of bytes following (min.=8)

02H   BYTE       model       PC XT PCjr AT PS/50/60 XT/2 PS/30 CVT PS/80

FF FE FD   FC          FB   FA    F9  F8

03H   BYTE       submodel                     04 05 01   00    00  00

04H   BYTE       BIOS_rev    0 for first release, 1 for 2nd, etc.

05H   BYTE       featbyte   80 = DMA channel 3 used by hd BIOS

40 = 2nd 8259 installed

20 = Real-Time Clock installed

10 = Int 15h, AH=41h called upon Int 9 (keyboard

scan code hook)

8 = wait for external event supported

(Int 15h, AH=41h) (CVT); reserved on PS/2

4 = extended BIOS area allocated at 640K

2 = bus is Micro Channel instead of PC

1 = reserved

06H   WORD       0  reserved

08H   WORD       0  reserved

----------------------------------------------------------------------------

CPU Global Descriptor Table:

-----------------------------------------------------------------------------

The global descriptor tables are used to perform block moves in  the

protected mode of the 80286/386 with Int 15h, function 87h, and for

switching to virtual mode, function 89h.

GDT organization:

offset 0  null descriptor

8  GDT descriptor

10  IDT descriptor

18  DS

20  ES

28  SS

30  CS

38  uninitialized, used to build descriptor for BIOS CS

Sample descriptor:

SOURCE_TARGET_DEF   STRUC

SEG_LIMIT      DW     ?   ; Segment limit (1 to 65536)

LO_WORD        DW     ?   ; 24-bit segment physical address (0 to 16M-1)

HI_BYTE        DB     ?   ;

DATA_ACC_RIGHT DW    93H  ; Access rights byte (CPL0-R/W)

Reserved       DW     0   ; Reserved word (must be 0)

SOURCE_TARGET_DEF   ENDS

Global descriptor table pointed to by ES:SI in Int 15h, function 87h:

BLOCKMOV_GDT_DEF    STRUC

DW     0,0,0,0  ; First dummy descriptor

CGDT_LOC       DW     ?,?,?,0  ; Location of calling routine GDT

SOURCE         DW     ?,?,?,0  ; Source descriptor - segment to be moved

TARGET         DW     ?,?,?,0  ; Target descriptor - destination segment

BIOS_CS        DW     ?,?,?,0  ; BIOS code descriptor - protected mode CS

TEMP_SS        DW     ?,?,?,0  ; Stack descriptor - protected mode SS

BLOCKMOV_GDT_DEF    ENDS

_____________________________________________________________________________

-----------------------------------------------------------------------------

BIOS Video Structures:

-----------------------------------------------------------------------------

Screen Attribute Byte:

-----------------------------------------------------------------------------

In alphanumeric modes (0-3 and 7), character data is stored in Map 0,

character attributes in Map 1, and character patterns in Map 2.  Map 0

and Map 1 can be viewed as a single buffer, with a character/attribute

pair stored at even addresses.

Bit  Function

7   Monochrome blinking or color background Intensity

6   Red background

5   Green background

4   Blue background

3   Foreground intensity or character select (see Char. Map Select Reg.)

2   Red foreground

1   Green foreground

0   Blue foreground

Mixed colors:   Non-intense:               Intense:

No color      = Grey

Green + Blue  = Cyan       Green + Blue  = Light Cyan

Red   + Blue  = Magenta    Red   + Blue  = Light Magenta

Red   + Green = Brown      Red   + Green = Yellow

All three     = High intensity white

-----------------------------------------------------------------------------

Screen Dynamic Video State Table:

-----------------------------------------------------------------------------

This table is returned in a 64K byte buffer using Int 10h, function 1Bh,

'Get Video BIOS Functionality/State Information'.

Offset  Data type  Description

00H    DWORD     Address of static video functionality table

04H    BYTE      Video mode

05H    WORD      Number of displayed character columns

07H    WORD      Length of displayed portion of video buffer in bytes

09H    WORD      Start address of upper left corner of video buffer

0BH    16 BYTES  Table of cursor locations (col.,row) for 8 video pages

1BH    BYTE      Cursor end line

1CH    BYTE      Cursor start line

1DH    BYTE      Active video page

1EH    WORD      I/O port for CRTC address register

20H    BYTE      CRT_MODE_SET (current value of 3x8H register)

21H    BYTE      CRT_PALETTE (current value of 3x9H register)

22H    BYTE      Number of displayed character rows

23H    WORD      POINTS (height of displayed character matrix)

25H    BYTE      Active display combination code

26H    BYTE      Inactive display combination code

27H    WORD      Number of displayed colors (0 for monochrome)

29H    BYTE      Number of video pages supported

2AH    BYTE      Raster scan lines: 0=200,1=350,2=400,3=480

2BH    BYTE      Alphanumeric character table used when

attribute bit 3 is 0 (VGA) 0=Block0,...0FFh=Block255

2CH    BYTE      Alphanumeric character table used when

attribute bit 3 is 1 (VGA) 0=Block0,...0FFh=Block255

2DH    BYTE      Misc. state information (1 if true):

Bit 0= all modes active on all video subsytems

1= gray-scale summing enabled

2= monochrome display attached

3= default palette loading disabled

4= cursor emulation enabled

5= blinking attribute enabled

6=7 reserved

2EH    BYTE      reserved

30H    BYTE      reserved

31H    BYTE      Video RAM available: 0=64K,1=128K,2=192K,3=256K

32H    BYTE      Save area status (1 if true):

Bit 0= two alphanumeric char. sets active (VGA)

1= dynamic save area is active

2= alphanumeric char.set override is active

3= graphics char.set override is active

4= palette override is active

5= display combination code extension active

6-7 reserved

33H-33FH         Reserved

----------------------------------------------------------------------------

Screen Static Video Functionality Table:

-----------------------------------------------------------------------------

The address for this table appears as the first double word in the

dynamic video state table.  The dynamic video table is returned in a 64K

byte buffer using Int 10h, function 1Bh, 'Get Video BIOS

Functionality/State Information'.

Offset Data type  Description

00H    BYTE      Video modes supported (1 if supported)

Bits 0-7=modes 0-7

01H    BYTE      Video modes supported

Bits 0-7=modes 8-15

02H    BYTE      Video modes supported

Bits 0-3=modes 16-19; bits 4-7 reserved

03-06H  4 BYTES   Reserved

07H    BYTE      Scan lines available in alphanumeric mode (1 if avail.)

Bit 0=200,1=350,2=400 lines

08H    BYTE      Maximum number of displayable alphanumeric char.sets

09H    BYTE      Number of available alphanumeric character definition

tables in char. generator ROM

0AH    BYTE      Misc. video BIOS capabilities (1 if available)

Bit 0= all modes on all monitors (0 for MCGA)

1= Gray-scale summing

2= char.set loading

3= default palette loading

4= cursor emulation

5= 64-color palette

6= video DAC loading

7= control of video DAC via attrib. controller

color select

0BH    BYTE      Misc. video BIOS capabilities (1 if available)

Bit 0= light pen support

1= save/restore video state

2= blinking/background intensity

3= display combination code

4-7 reserved

0CH    BYTE      Reserved

0DH    BYTE      Reserved

0EH    BYTE      Save area capabilities

Bit 0= multiple alphanumeric char.sets

1= dynamic save area

2= alphanumeric char.set override

3= graphics character set override

4= palette override

5= Display Combination Code extension

6-7 reserved

0FH    BYTE      Reserved

----------------------------------------------------------------------------

Screen Alphanumeric Character Pattern Table:

-----------------------------------------------------------------------------

The alphanumeric character pattern table addressis stored in the Int 1Fh

vector.

This is a pointer to graphic characters to replace 80H-FFH (8x8 matrix

read from top to bottom to make 8 bytes per character). Used in color

graphics mode.

_____________________________________________________________________________

-----------------------------------------------------------------------------

MS-DOS Structures:

----------------------------------------------------------------------------

Disk Parameter Block (DPB):

-----------------------------------------------------------------------------

The disk parameter block is an internal parameter block used by MS-DOS,

and is built from the BIOS Parameter Block (BPB).  Function 32h under

Int 21h can be used to locate the disk parameter block.  A different

disk parameter block can be generated for each unit of a block device

driver, supporting different media.

The DOS disk partition length limitation of 32 Megabytes comes from the

fact that MS-DOS uses WORD storage for logical sector pointers.  Each

sector is nominally 512 bytes.  This limitation is shown in DOS

interrupt functions Int 25h and Int 26h.

Internal Disk Parameter Block (built by MS-DOS from BPB)

Offset  Data type Description

00H    BYTE      Drive Index (0='A', etc.)

01H    BYTE      Unit code: unit number in device driver

02H    WORD      Number of bytes/sector

04H    BYTE      Sectors/cluster - 1

05H    BYTE      Log base 2 of sectors/cluster

06H    WORD      Reserved sectors

08H    BYTE      Number of FAT copies

09H    WORD      Number of root DIR entries

0BH    WORD      Sectors in reserve, FATs, and DIR: First data sector

0DH    WORD      Max number of clusters + 1 available above DIR

0FH    BYTE      Sectors per FAT (WORD in DOS 4.x, shifting remaining

addresses below by one byte.)

10H    WORD      Sectors in reserve and FATs: First DIR sector

12H    DWORD     Pointer to device driver for this device

16H    BYTE      Media descriptor byte (Disk ID byte)

17H    BYTE      Validity byte: 0FFH means block must be rebuilt

18H    DWORD     Address of next device driver or -1 if last

1CH    WORD      Starting cluster of current directory (0 for root)

(DOS 2.x) or cluster to start search for next free

cluster (DOS 3.x and 4.x)

1EH    64 BYTES  ASCIZ string for current directory path (DOS 2.x)

or 0FFFFH (DOS 3.x)

----------------------------------------------------------------------------

File Allocation Table (FAT) Structure:

-----------------------------------------------------------------------------

12-bit FAT layout (for DOS 1.x, 2.x, and 3.x if partition is less than

16 Megabytes, i.e. total clusters is less than 4087):

Tribble Data type Description (values in hex)

Offset

0     TRIBBLE   Disk ID byte + F00

1     TRIBBLE   Filler = FFF

2     TRIBBLE   Cluster Value:  000 = unused

001-FEF = next cluster number

FF0-FF6 = reserved cluster

FF7 = cluster marked bad

FF8-FFF = last cluster in file

003H   TRIBBLE   Etc.

16-bit FAT layout (for DOS 3.x if partition is greater or equal to

16 Megabytes, i.e. total clusters is 4087 or greater):

Word   Data type Description (values in hex)

Offset

0     WORD      Disk ID byte + FF00

1     WORD      Filler = FFFF

2     WORD      Cluster Value:   0000 = unused

0001-FFEF = next cluster number

FFF0-FFF6 = reserved cluster

FFF7 = cluster marked bad

FFF8-FFFF = last cluster in file

For example, the first file disk directory entry might have a starting

cluster of 0002.  At entry 2 in the FAT, there may be a next cluster

value of 0003; at 3 there may be 0008; at 8 one might find FFF8,

indicating this indexed position is the last cluster in the file.

----------------------------------------------------------------------------

Disk Directory Entry: (See also FCB)

-----------------------------------------------------------------------------

Every formated disk has a root directory where information about files

and subdirectories is kept.  The root directory begins just after the

file allocation tables.  Its starting sectory on the disk can be found

from the BIOS parameter block on the disk's boot sector.  Subdirectories

are stored on disk just like files, with their own cluster assignments.

The first two file structures in a subdirectory have a dot and a double

dot in the filename field, representing the current directory and the

parent directory.

Offset  Data type Description

00H    BYTE      First character of filename, or

E5H  if file has been erased, or

00H  if directory entry has not been used (there

are no more used entries in directory), or

05H  if first character of file is to be 0E5H, or

2EH = '.' if this entry is an alias for the current

or parent directory.  If next byte is a space (20H),

the cluster field gives the starting cluster for

the current directory.  If the next byte is also a

dot (2EH), then cluster field has the starting

cluster of the parent or zero if the parent is the

root directory.

01H    10 BYTES  The next 7 characters of the filename and the

file extension.  Filled with 20H = ' '.

0BH    BYTE      File attribute:

Bit 7-6 Reserved

5  Archive (set whenever file is changed)

4  Subdirectory

3  Volume label (only in root directory)

2  System file

1  Hidden file

0  Read only (file writes denied)

0CH    10 BYTES  Reserved

16H    WORD      Time created or last updated

Bit 15-11  Binary number of hours (0-23)

10- 5  Binary number of minutes (0-59)

4- 0  Binary number of even seconds (0-29)

18H    WORD      Date created or last updated

Bit 15- 9  Binary number of years since 1980

8- 5  Binary number of months (1-12)

4- 0  Binary number of days (1-31)

1AH    WORD      Starting cluster

1CH    DWORD     File size

-----------------------------------------------------------------------------

Disk File Control Block (FCB): (See also Disk Directory Entry)

-----------------------------------------------------------------------------

The file control block is a storage buffer for transferring information

about a current file to and from the disk operating system.  Before the

introduction of file handles (i.e. before DOS 2.x), the user was

required to set up his own FCBs for each file accessed.  The structure

of a FCB is:

Offset  Data type Description

-07H    BYTE      0FFH = Extended FCB flag indicating seven extra bytes

-06H    5 BYTES   Reserved

-01H    BYTE      Attribute of file:

Bit 5 Archive

4 Subdirectory

3 Volume Label

2 System file

1 Hidden file

0 Read-only file

00H    BYTE      Drive identification: 0=default, 1='A', etc.

01H    8 BYTES   Filename              space filled.

09H    3 BYTES   File Extension        space filled.

0CH    WORD      Current block         (1 block = 128 records)

0EH    WORD      Record size           (default = 80H = 128 bytes)

10H    DWORD     File size

14H    WORD      Date created/updated  (8 bit:year-1980, 5 bit month, day)

16H    WORD      Time created/updated  (5 bit hour, 6 bit min, even secs)

18H    8 BYTES   Reserved              (contains device driver pointer)

20H    BYTE      Current record        Offset of record in block

----------------------------------------------------------------------------

Country Information Table:

-----------------------------------------------------------------------------

The country information table location can be found by using function

38h, Int 21h.

Country Information Table

Offset  Data type Description

00H    WORD      date format (see above)

02H    5 BYTES   ASCIIZ string showing currency country symbol

07H    2 BYTES   ASCIIZ string showing thousands separator character

09H    2 BYTES   ASCIIZ string showing decimal separator character

0BH    2 BYTES   ASCIIZ string showing date separator character

0DH    2 BYTES   ASCIIZ string showing time separator char

0FH    BYTE      currency format

bit 1 = number of spaces between value and currency symb.

bit 0 = 0 if currency symbol precedes value

1 if currency symbol follows value

10H    BYTE     Number of digits after decimal in currency

11H    BYTE     Time format

bit 0 = 0 if 12-hour clock

1 if 24-hour clock

12H    DWORD    Address of case map routine (FAR CALL) for codes >7Fh

Call with AL=ASCII code to be converted, AL=return.

16H    BYTE     Data-list separator char

17H    BYTE     Null = 00H

18H    10 BYTES  reserved

Time and Date format:

0 = USA standard    h:m:s m/d/y

1 = Europe standard h:m:s d/m/y

2 = Japan standard  h:m:s y:m:d

----------------------------------------------------------------------------

Device Driver Device Information Word:

-----------------------------------------------------------------------------

The device information word is passed to a device driver through Int

21h, function 44h (IOCTL).  Its setting is very similar to the third

word in the device driver header.

Bits  15  = 0  reserved

14  = 1  if can handle control codes

13-8 = 0  reserved

7   = 1  character device

= 0  block device

6   = 1  EOF on input

Character Device:                  Block Device:

5   = 1  'raw' mode               Bits 5-0 = drive index

= 0  'cooked' mode                       (0='A', 1='B', etc.)

4   = 0  reserved

3   = 1   clock

2   = 1   null device

1   = 1   console output device

0   = 1   console input device

----------------------------------------------------------------------------

Device Driver Header:

-----------------------------------------------------------------------------

The device driver header appears as the first block of data in a device

driver.  Device drivers are installed by the operating system, starting

with the NUL device driver.  MS-DOS contains default device drivers for

the console (CON:), a logical printer (PRN:), an auxiliary device

(AUX:), serial devices (COMx:), parallel devices (LPTx:), and a system

clock (`CLOCK).  Replacement or supplemental drivers can be added using

device driver file names in the CONFIG.SYS file.  Note that a device

drive file name may not be the name needed to refer to the device in a

device open call.  The device name appears at offset +0AH in the device

driver header.

Offset  Data type  Description

00H    DWORD     Pointer to next device or -1

04H    WORD      attribute:

bit: 15 = 1 if character device, 0 if block device

14 = 1 if IOCTL (can handle control strings)

13 = if block device, 1 = non-IBM format

if char. device, 1 = supports output-until-busy

12 = 0 reserved

11 = 1 if device supports OPEN/CLOSE/REMOVABLE

7-10 = 0 reserved

6 if supports Get/Set Logical Device

5 if char. device, 1='raw' mode, 0='cooked' mode

4 if char. device, 1=Special

3 if char. device, 1=Current CLOCK$

2 if char. device, 1=Current NUL Device

1 if char. device, 1=Current Standard Output

0 if char. device, 1=Current Standard Input

06H    WORD      pointer to strategy entry

08H    WORD      pointer to interrupt entry

0AH    8 BYTES   If char. device, name field

If block device, 1 byte unit number

Communication with device drivers is usually performed by the operating

system through request packets.

----------------------------------------------------------------------------

Device Driver Device Request Header and Device Request Packet:

-----------------------------------------------------------------------------

The device request header is used by the operating system to communicate

with a particular device driver.  It forms the beginning of the device

request packet, a memory structure described below:

Offset in device request header:

For all functions:

+ 0  byte length

+ 1  byte unit code ('minor' device index)

+ 2  byte command code

Extended Offset for device request packet:

function 0 init :

+ D byte number of units

+ E dword break addr

+12 dword BPB array

function 1 media chk :

+ D byte media desc. from BPB

+ E byte returned

function 2 build BPB :

+ D byte media desc. from BPB

+ E dword transfer addr. (128 bytes) or FAT

+12 dword BPB

function 3,12 IOCTL in out, 4 read, 8,9 write, with verify

+ D byte media desc. from BPB

+ E dword transfer addr

+12 word byte/sector count

+14 word starting sector number

function 5 non-dest. read

+ D byte read from device

function 6 in status, 10 output status

function 7 input flush, 11 output flush

+ 3  word status (0 on entry)

bit 15=1=>ERROR in bits 7-0

bit  9=1 device not busy on status call

bit  8=1 device operation complete

+ 5  Dword to ms-dos queue

+ 9  Dword to device queue

----------------------------------------------------------------------------

Memory Control Block:

-----------------------------------------------------------------------------

The memory control block is a 16 byte prefix to all programs and each

copy of the environment.  It is used by the operating system to keep

track of memory allocation, and can be found 16 bytes below the Program

Segment Prefix.  The first MCB can be found using function 52h of Int

21h.

Memory Control Block structure:

Offset  Data type Description

00H    BYTE      4DH if in chain, 5AH if end of chain ('MZ'=Mark Zbikowski)

01H    WORD      0 if free or Program owner PSP segment word

03H    WORD      size (in paragraphs) of controlled memory arena

----------------------------------------------------------------------------

Environment Block:

-----------------------------------------------------------------------------

The environment blocks reside in memory just before each child process,

unless they have been deallocated by a resident program.  The original

environment is located just after the COMMAND processor.  It can be

found by using function 52h of Int 21h to get the first memory control

block, then jumping through the MCBs to locate the desired environment

block.

The environment blocks contain the definitions of variables used by DOS

and by application programs.  For example, COMSPEC, used in the

CONFIG.SYS file, and PATH and PROMPT, used in the AUTOEXEC.BAT file, are

entered into the original environment as variables.  Moreover, the SET

command defines user variables in the environment.

Environment Block Structure:

Offset  Data type Description

00H    n1 BYTES  ASCIIZ string in form: 'PARAMETER1=VALUE1',0

n1+1   n2 BYTES  ASCIIZ string in form: 'PARAMETER2=VALUE2',0

etc.

nz     BYTE      0  end of variable definition strings

nz+1   WORD      Count of number of following ASCIIZ strings (DOS 3.xx)

nz+3   ns BYTES  Name of Current Process as ASCIIZ string (DOS 3.xx)

Note that variable definitions end with a double null.  The current

process ASCIIZ string contains the path and filename for the program

residing in memory which owns the environment block.  This information

was not available before DOS 3.xx.

----------------------------------------------------------------------------

Program Segment Prefix:

-----------------------------------------------------------------------------

Before any program is run, MS-DOS sets up a 256 byte program segment

prefix containing termination vectors, default file control blocks, and

the tail of the command line used to start the program.  (This structure

is a carry-over from the CP/M operating system.)  The file control

blocks are filled in with one or two file names if these are found in

the correct format in the command line tail.

PSP Structure:

Offset  Data type Description

00H  WORD        INT 20H

02H  WORD        end of allocation block segment (top of memory)

04H  2 BYTES     reserved

06H  WORD        number of bytes available in this segment

08H  2 BYTES     reserved

0AH  DWORD       contents of INT 22H termination vector

0EH  DWORD       contents of INT 23H ^C break vector

12H  DWORD       contents of INT 24H critical error vector

16H  WORD        parent PSP segment

18H  BYTE        open files (0FFH=unused)

19H  19 BYTES    reserved

2CH  WORD        segment of environment block

2EH  DWORD       old stack address

32H  BYTE        maximum open files

34H  DWORD       old file address

36H  25 BYTES    reserved

50H  WORD        long call address to DOS INT 21H handler

5CH  16 BYTES    default FCB #1

6CH  16 BYTES    default FCB #2

80H  128 BYTES   command tail and default DTA

----------------------------------------------------------------------------

DOS Driver List:

-----------------------------------------------------------------------------

The DOS drive list is reference data for device drivers kept in memory

by  the operating system. It is an undocumented internal data area which

is strongly version dependent. A pointer to this list is returned by the

internal function 52h under Int 21h.

Offset  Data type Description

00H    DWORD     Pointer to first disk parameter block

(each disk parameter block contains pointer to next one)

04H    DWORD     Pointer to internal data area

08H    DWORD     Pointer to CLOCK$ device driver

0CH    DWORD     Pointer to actual CON: device driver

If DOS 2.x:

10H    BYTE      Number of logical drives in system

11H    WORD      Maximum bytes/block of any block device (512)

13H    DWORD     Pointer to current internal disk transfer area

17H    n BYTES   Beginning of NUL device driver.

If DOS 3.x:

10H    WORD      Maximum bytes/block of any block device (0200h)

12H    DWORD     Pointer to current internal disk transfer area

16H    DWORD     Pointer to drive dynamic information array:

Drive Dynamic Information Array (81 bytes per drive, starting

with drive A:

00H  69 BYTES Current path as ASCIIZ string, starting with 'x:\'

45H  DWORD    Pointer to disk parameter block for this drive

49H  WORD     Reserved

4BH  DWORD    Reserved

4FH  DWORD    Reserved

1AH     DWORD     Pointer to data area

1EH     WORD      Reserved

20H     BYTE      Number of block devices

21H     BYTE      Value of LASTDRIVE command in CONFIG.SYS (default 5)

22H     n BYTES   Beginning of NUL device driver.

-----------------------------------------------------------------------------

Font Files and Code Pages Images:

-----------------------------------------------------------------------------

Code page images are contained in font files (extension .CPI). These can

be downloaded to the device, if the device has a code page switching device

driver entry in CONFIG.SYS.

Data type  Value  Description

file_header:

8 BYTES   ?      0FFH,"font   "  Font tag (7 characters)

8 BYTES   0      Reserved

WORD      1      Number of pointers in header (1 for DOS 3.3)

WORD      1      Type of info pointer (1 in DOS 3.3)

2 WORDS   ?      Offset info, 0  Displacement of info in file

info:

WORD      1      Number of code page entries

code_page_header:

WORD      ?      Size of this code page header

2 WORDS   0,0    Pointer to header of next code page

WORD      1      Device type: 1 = display, 2 = printer

8 BYTES   ?      "EGA     "  Device sub type ID

WORD      ?      999  Code page ID

3 WORDS   0      Reserved

data_header:

WORD      1      Reserved (must be 1)

WORD      ?      Number of fonts

WORD      ?      Length of following font data

If the font file is for a display device, then:

font_1:

BYTE      ?      character size, rows

BYTE      ?      character size, columns

2 BYTES   0,0    aspect ratio

WORD      256    number of characters

...

pixel description for all 256 characters

...

font_2:

...

font_3:

...

Else, if font file is for a printer, then:

selection_type_1:

WORD      1      selection type (1=4201, 2=5202 Proprinter)

WORD     12      total number of bytes in control sequences

6 BYTES   5,27,"I",0,27,"6" defines hardware code page

6 BYTES   5,27,"I",4,27,"6" defines downloadable code

...              character description of download

selection_type_2:

WORD      2      selection type

WORD     12      total number of bytes in control sequences

6 BYTES   27,"[","T",5,0,0,0,1,181,0,26,"6"  select active page 437

...              character description of download

_____________________________________________________________________________

-----------------------------------------------------------------------------

DOS File Formats:

-----------------------------------------------------------------------------

File EXE Program Header:

-----------------------------------------------------------------------------

All .EXE files on disk begin with a header to determine how the segment

and pointer registers are to be initialized, and to relocate absolute

addresses.

EXE header and program structure:

Offset  Data type Description

00H    BYTE      4DH signature byte 1

01H    BYTE      5AH signature byte 2 (for Mark Zbikowski)

02H    WORD      file size modulo 512

04H    WORD      file size in pages, including header

06H    WORD      number of relocatable table items

08H    WORD      size of header in paragraphs

0AH    WORD      Min. number of paragraphs needed above program

0CH    WORD      Max. number of paragraphs desired above program

0EH    WORD      segment displacement of stack module

10H    WORD      contents of SP at entry

12H    WORD      word checksum

14H    WORD      contents of IP register at entry

16H    WORD      segment of displacement of code module

18H    WORD      offset of first relocation item in file

1AH    WORD      overlay number (0 for resident part of program)

1CH    n BYTES   reserved space

relocation table

reserved space

program and data segments

stack segment

-----------------------------------------------------------------------------

MicroSoft Object Module Structure:

-----------------------------------------------------------------------------

Object modules are files created by language compilers as an intermediate

step between source code and machine code.  Individually, they have

a .OBJ extension to their name.  Collectively, in libraries, they have

a .LIB extension.

Each object module contains a sequence of variable-length records.

Each record has the following format:

1 BYTE  Record Type (80H to 0B0H)

1 WORD  Record Length (for next n+1)

n BYTES Record Body

1 BYTE  Checksum (Sum of all n+4 bytes = 0 mod 256)

Record Types:

(Index Fields coded as: Actual length if < 128 size

81H , 2-byte size

84H , 3-byte size

88H , 4-byte size)

80H THEADR Translator Header Record (always first record of .obj file)

The record body, the T-module name, is a variable-length field

containing:

1 BYTE  The length of the name of the object module

n BYTES The name of the object module defined by NAME in MASM

88H COMENT Comment Record (never between FIXUPP and LEDATA or LIDATA)

Record body:

1 BYTE  Attribute: Bit 7=1 No Purge: Utilities should preserve comment.

6=1 No List:  Utilities should not list comment.

5-0 0

1 BYTE Comment Class: 0   Language-translator name given by comment.

1   Copyright given by comment.

2-9BH Reserved by Intel

9CH MS-DOS version number.

9DH Memory model: s,c,m,l, and h for

small, compact, medium, large, and huge

9EH Sets Microsoft LINK's DOSSEG switch.

9FH Default library search name.

A1H Microsoft extensions to Intel object

record specifications are used.

C0H-FFH Reserved.

n BYTES Comment Field

8AH MODEND Module End Record (Always last record in .obj file)

Record body:

1 BYTE Module type: Bit 7=1 if module is a main program

6=1 if if this record contains a START address

6-1 0

0=1 START address is relocatable (always

for 8086 code)

n BYTES START address field (when Module type bit 6=1)

Format: EndDat, FrameDat, TargetDat, TargetDispl.

(See FIXUPP record for interpretation.)

8CH EXTDEF External Names Definition Record

Record body:

1 BYTE  Name Length (n= 0-7FH)

n BYTES External Name

1 BYTE  Type index: TYPDEF record index associated with External Name

(unless 0).

8EH TYPDEF Type Definition Record (Precedes EXTDEF or PUBDEF records which

reference a TYPDEF)

Record body:

1 BYTE  Name field (0 for Microsoft TYPDEF)

n BYTES Intel Eight-Leaf Descriptor field (Microsoft uses only four)

1 BYTE  0

m BYTES Leaf Descriptor: Four Fields ("Leaves") to describe

the type and size of a variable:

1 BYTE  Variable Type

= 62H if NEAR Variable

= 61H if FAR  Variable

1 BYTE  Variable Subtype (ignored by LINK)

= 77H Array

79H Structure (NEAR variable only)

7BH Scalar (NEAR variable only)

i BYTES Size of communal variable (if NEAR) or

Number of elements of array (if FAR)

as an Index Field

t BYTES For FAR array only: Index of previous

NEAR TYPDEF containing the array data

type and size of elements in array;

Index of 1 means first TYPDEF, etc.

90H PUBDEF Public Names Definition Record (must be after GRPDEF and SEGDEF

records to which it refers; PUBDEF usually placed near end of

object module).

Record Body:

n BYTES Public Base

i BYTES Group Index Field references a previous GRPDEF record

(0 means no group associated with this PUBDEF)

i BYTES Segment Index Field references a previous SEGDEF record

(if 0, Frame Number used and Group Index = 0)

1 WORD  Frame Number (only if GI and SI = 0). Specifies

the start of an absolute segment.  Value indicates

the number of the frame containing the public name.

m BYTES Public Name

1 BYTE  Length of name (< 128)

p BYTES Name

1 WORD  Public Offset of location referred to by Public

Name.  Offset assumed to lie within segment, group,

or frame specified in the public base field.

i BYTES Type Index

Index Field references a previous TYPDEF.

...

Above three fields can be repeated to declare a list of public names.

94H LINNUM Line Number Record relates line numbers in source to

addresses in object code. (Must be after SEGDEF to which

they refer.  LINNUM records generally placed near end

of object module).

Record body:

l BYTES Line Number Base

1 BYTE  Group Index = 0

i BYTES Segment Index. Index Field references previous

SEGDEF record.

1 WORD  Line Number (0 to 32,767)

1 WORD  Line Number Offset in executable code in segment specified

in the Line Number Base field to which the line number in

the Line Number field refers.

...

Above two fields can be repeated, so that a single LINNUM record

can specify multiple line numbers in the same segment.

96H LNAMES List of Names Record.  List of names that can be referenced

by subsequent SEGDEF and GRPDEF records.  LNAMES records usually

placed near start of object module.

Record Body:

n BYTES Name List

1 BYTE  Length of name (0-255)

l BYTES Name

... up to 255 logical names, implicitly index 1,2,...

98H SEGDEF Segment Definition Record describes a logical segment in an

object module, giving the segment's name, length, alignment, and

the way the segment can be combined with other logical segments.

LINK limits the number of SEGDEFs to 255 per object module.

Record body:

n BYTES Segment Attributes

1 BYTE  ACBP byte:

Bits 7-5  Alignment in the run-time memory map

= 0 A = Absolute

= 1 Relocatable, byte (1) aligned

= 1 Relocatable, word (2) aligned

= 1 Relocatable, paragraph (16) aligned

= 1 Relocatable, page (256) aligned

4-2  C = Combination with other segments

= 0 if A = 0 or private segment

= 1 not used by Microsoft

= 2 can be concatenated with another segment

with the same name; used with 'public'

combine type

= 3 undefined

= 4 same as C = 2 for Microsoft

= 5 can be concatenated with another segment

with the same name; used with 'stack'

combine type

= 6 can be overlapped with another segment

with the same name; used with 'common'

combine type

= 7 save as C = 2 for Microsoft

1  B = Big

= 1 (and Segment Length = 0) only if the

segment is exactly 64K long

0  P = Page-resident (not used in MS-DOS)

= 0 for Microsoft

x BYTES Frame Number and Offset (present only for

A = 0 in ACBP byte)

1 WORD  Frame Number for start of segment

1 BYTE  Offset (0-0FH) Ignored by LINK

1 WORD  Segment Length in bytes (0-0FFFFH).  If segment is

exactly 64K, Segment Length = 0 and B = 1 in ACBP.

i BYTES Segment Name Index Field identifies the segment

with a unique name.

i BYTES Class Name Index Field identifies the segment with a

class name (such as CODE, FAR_DATA, or STACK).  The

linker places segments with the save class name into

a contiguous area of memory in the run-time memory

map.

i BYTES Overlay Name Index Field identifies the segment with a

run-time overlay.  Ignored by LINK ver. 2.40 and later.

9AH GRPDEF Group Definition Record defines a group of segments, all

of which lie within the same 64K frame in the run-time memory map.

LINK limits number of GRPDEF to 21.

Record body:

i BYTES Group Name Index Field indicates the name in LNAMES record.

n BYTES Group Component Descriptor

1 BYTE  Type = 0FFH means following field is Segment Index.

i BYTES Segment Index Field identifies previous SEGDEF record.

...

Group Component Descriptor is usually be repeated.

9CH FIXUPP Fixup Record contains information to allow linker to

resole addresses.  FIXUPP records describe the location of

each address value to be fixed up, the target address to

which the fixup refers, and the frame relative to which the

address computation is performed.  FIXUPP must immedately

follow LEDATA or LIDATA record if it refers to one of these.

Record body:

n BYTES Thread Field. FIXUPP may contain 0 or more Thread Fields.

Format:

1 BYTE  Thread Data

Bits 7   = 0 Start of thread field

6   = 1 if thread specifies a frame

0 if thread specifies a target

5   = 0 unused

4-2 = Method:

Frame specified by:

0 a segment index

1 a group index

2 an external index

3 explicit frame number (not supported by LINK)

4 the segment in which the Location is defined;

the largest possible frame number is used.

5 the Target's segment, group, or external index.

Target specified by:

0 a segment index and a displacement.

The displacement is given in the Target

Displacement field of the FIXUPP record.

1 a group index and a Target Displacement.

2 an external index and a Target Displacement.

LINK adds the displacement to the address it

determines from the external name's corresponding

PUBDEF record in another object module.

3 an explicit frame number (not supported by LINK).

4 a segment index alone (displacement = 0)

5 a group index alone

6 an external index.  The Target is the address

associated with the external name.

7 an explicit frame number (not supported by LINK).

1-0 = Thread Number (0-3) for Frame or Target. The

thread number can be reused in an object module;

if so, it refers to the thread field in which

it last appeared.

i BYTES Thread Index Field:

If Method = 0 : Segment index in previous SEGDEF record

1 : Group index in previous GRPDEF record

2 : External index in previous EXTDEF record

3 : Explicit Frame number (not supported by

LINK; ignored)

n BYTES Fixup Field; provides information needed to resolve a

reference to a relocatable or external address.  FIXUPP may

contain 0 or more Fixup Fields.

Format:

1 WORD Locat field:

Bits:    7    = 1  indicates fixup field

6    = M = 1 if fixup is segment relative

0 if fixup is self-relative

5    = S = 0 unused

4-2    = Loc field for type of Location to fix:

0 = Low-order byte

1 = Offset

2 = Segment

3 = Pointer (Segment:offset)

4 = High-order byte (not recognized

by LINK)

5 = Loder-resolved offset (treated

as Loc=1 by LINK)

1-0,15-8 Data Record Offset; indicates the

position of the Location to be fixed

in LEDATA or LIDATA immediately preceding

the FIXUPP record.  This offset indicates

either a byte in the Data field of an

LEDATA record or a data byte in the Content

field of an Iterated Data Block in an

LIDATA record.

1 BYTE  Fix Dat:

Bit   7  = F = 1 if Frame for this fixup is specified

by reference to a previous Thread Field;

= 0 if the Frame method is explicitly

defined in this fixup field.

6-4 = Frame:

If F=1, Frame = number (0-3) of thread field

containing the Frame method;

F=0, Frame = Thread Field Method number

(0-5)

3 = T = 1 if Target for fixup is specified by

a reference to a previous Thread field

0 if the Target is explicitly defined in

this fixup field

2 = P bit; determined by 1-0 bits:

1-0 = Targt field: If T=1, Targt contains the

number of the previous Thread field containing

the Target method and P bit combined with

the 2 low-order bits of the Method field in

the Thread field determines the Target method;

If T=0, target is explicitly defined and the

P and Targt fields contain number (0-6) of

one of the Target fixup methods under the

Thread Field method bits for Target.

i BYTES Frame Datum is an index field that refers to a previous

SEGDEF, GRPDEF, or EXTDEF record, depending on the

Frame method.

i BYTES Target Datum contains a segment index, a group index,

or an external index field depending on the Target

method.

1 WORD  Target Displacement present only if the P bit in

the Fixdat field is 0.  If so, Target Displacement

contains the offset used in methods 0-2 specifing

a Target.

A0H LEDATA Logical Enumerated Data Record

The LEDATA record contains contiguous binary data, executable code

or program data, that is eventually copied into the program's

executable binary image.

Record body:

i BYTES Segment Index Field refers to a previous SEGDEF to which

data is to be placed.

A2H LIDATA Logically Iterated Data Record

The LIDATA record contains binary data (code or program data)

which can be specified by a repeating pattern rather than by

explicit enumeration.

Record body:

i BYTES Segment Index Field refers to previous SEGDEF to which

data is to be placed.

1 WORD  Iterated Data Offset into the segment referenced by the

Segment Index; relative to the base of the segment;

determines where the enumerated data will be placed in

the run-time memory map.

d BYTES Iterated Data Block (may be nested to contain one or

more other iterated data blocks):

1 WORD  Repeat Count for contents field

1 WORD  Block Count for number of iterated data blocks

in content field

n BYTES Content: Either nested iterated data blocks

if the Block Count is nonzero, or data; if

data:

1 BYTE  Number of data bytes in field

m BYTES Actual data

B0H COMDEF Communal Names Definition Record

The COMDEF record is a MicroSoft extension to the basic set

of Intel record types; recognized by LINK 3.50 and later.

Record body (can be repeated):

c BYTES Communal Name

1 BYTE  Length of name

n BYTES Name

i BYTES Type Index Field references a previous TYPDEF record

in the object module, or 0 if no public name associated.

1 BYTE  Data Segment Type

= 61H  FAR

62H  NEAR

n BYTES Communal Length indicates the amount of memory to be

allocated for the communal variable.

If Data Segment Type is NEAR, the Communal Length

contains the size (in bytes) of the communal variable;

If Data Segment Type is FAR, the Communal Length Field is:

i BYTES Number of Elements (Index Field) in communal

variable array of elements

i BYTES Element Size (Index Field) for each element

in the comunal variable array.

-----------------------------------------------------------------------------