Thank you for purchasing Crosstalk 3.0.  The cable needed for the

Molecular, the Altos 8000-2, and the Computpro Interfacer 4is given

below.  If you have another computer your cable diagram is listed

in the manual.

The Altos 8000-2 uses the same cable as the Altos Series 5 listed in

manual.

Molecular          Modem

w/AP8001 or AP8003

2 -------------- 3

3 -------------- 2

4 -------------- 8

7 -------------- 7

Molecular          Modem

w/AP8020 or AP8015

2 -------------- 3

3 -------------- 2

4 -------------- 8

5 -------------- 20

7 -------------- 7

Note - Make sure you choose the correct AP board from the install menu.

Also, the AP8001 and the AP8003 are only recommended for use with

the Hayes Smartmodem or compatibles.  Also, note that your modem

switches should be set so that DTR is always true(i.e.- switch 1

on the Hayes Smartmodems should be down).  This only applies to

the Molecular computers using the AP8001 or the AP8003.

Compupro           Modem

(Master mode)

1 -------------- 1

2 -------------- 2

3 -------------- 3

7 -------------- 7

8 -------------- 8

20 -------------- 20

Compupro           Modem

(Slave mode)

1 -------------- 1

2 -------------- 3

3 -------------- 2

6 -------------- 20

7 -------------- 7

8 -------------- 8