PCT3JR.DOC

PCT3JR.MRG

03/11/84  Michael Eorgoff

PPN  70516,1272

PCTJR.MRG is a "MERGE" file for PC-TALK3.  It corrects the activities

of the Alt-R(receive file) and Alt-S(print screen) in the PCjr environment.

The PCjr shuts off and ignores all other interrupts when it is time to do

a disk access.  For the Alt-R without any modifiers(ie.=x), XOFF is sent

to the remote computer whenever it becomes desirable for the PCjr to

access the disk.  The loop using 64060-64080 is used to wait for incoming

comm activity to die down.  All input from the comm buffers is put into

intermediate buffers until such time as there is no more.  Then the inter-

mediate buffers are written out to disk.  After the intermediate buffers

are written out, then an XON is sent to the remote computer.  Closing the

file is nested in a check for outstanding intermediate buffers that need

to be written before the actual close is performed.

The choice of five buffers for the intermediate buffering was

made by trial, error, and esthetics.  The first reason for multiple buffers

is that some transmitting programs are slow to react to the XOFF request.

PCTALK is one of these.  Also, at 1200 baud, there can be an appreciable

amount of information that is incoming in the transmission network being

employed.  What happens is that even though you have issued the XOFF

in your program, the remote computer may not immediately respond.  This

leaves the communications input buffer in a non-empty condition.  Therefore,

you must empty it into multiple buffers.  The XOFF may be slow in getting

transmitted.  It appears that BASIC handles all the incoming information

before worrying about the outgoing.  The esthetic consideration is also one

of performance. The screen display gets very choppy when small numbers of

buffers are used.  The amount of time overhead needed to wait out the

incoming characters, and the actual disk time, appear to require that more

buffering is supplied so that the communications doesn't appear to be

unreasonably slowed down.

The Alt-S(screendump) was couched in an XOFF-XON pair during the

entire duration of its activity.

Remember, is all else fails, go to sleep.  It won't get any better.

The manuals don't necessarily provide ALL the information, and probably most

of the books won't either.

P.S. PCT3JR was used to upload itself.