BSTAM from Lifeboat Associates

------------------------------

The BSTAM system distributed as TRANSMIT.COM and

RECEIVE.COM have been installed with interfaces

to specific UART/USART chips. The system for

S100 systems is supplied with routines for an

8251 USART addressed at ports 2/3. This is usable

without modification with the IMSAI 2SIO PORT A or

the North Star Horizon Left Serial Port. The source

of the routine is the file U8251.ASM.

To the Heath/TRS-80 user, the programs have installed

the TRS-80 interface. The source is U1602/43.ASM, and

is set up for 9600BD.

Other files are supplied with the system as further

examples of interfacing. The user should familiarize

himself with the I/O requirements of the system to

be equipped and edit and assemble an appropriate

I/O interface module. This is installed thus:

A> ASM UART	assemble UART.ASM, your own

interface

A> DDT RECEIVE.COM

I UART.HEX

R		read UART.HEX over RECEIVE.COM

^C		boot

A> SAVE 19 RECEIVE2.COM

save the patched file with a

temporary name for testing

A> DDT TRANSMIT.COM

I UART.HEX

R

^C		same again for transmit

A> SAVE 26 TRANSMI2.COM

The files RECEIVE2 and TRANSMI2 will now be customized

to your own I/O patches, and can be tested.

Don't forget that if two computers are taking to each other

along an RS232 cable set, one must appear as a data terminal

to the other. This normally involves reversing the leads

between pins 2 and 3 of one of the EIA connectors.

Machine 1		Machine 2

---------		---------

PIN 2       to	  PIN 3

PIN 3       to	  PIN 2

PIN 7       to          PIN 7

No other handshaking is required, since all of the acknowlegement

is performed in software.

#############