To:     All MS OEMS

From:   JCP MS 8087 Software Engineering

RE:     OEM specific MS software.

Date:   Aug 19, 1982

Revised Dec 7th, 1982

Revised Jul 18, 1983

The  3.11  beta  test  versions  of  the  Microsoft languages Pascal and

FORTRAN ( and all subsequent versions  utilizing  the  i8087  )  require

special software  provided  by  the  OEM.  The OEM software is necessary

to initialize the 8087 coprocessor and its interrupt handler.

Flow of control

---------------

The flow  of  control  between  Microsoft  software  and OEM software is

diagrammed below.

|       Microsoft                       OEM

|       ENTX??.ASM                      OEMR7.ASM

+---------------+                 +-------------+

|BEGXQQ:|       |                 |             |

|       |       |                 |             |

|       V       |                 |             |

|  +----------+ |                 |             |       OEMR7.ASM     INTR7.ASM

|  |  INIX87 ------------------------> OEMR87 --------> +---------+   +--------+

|  |    |     | |                 |      |      |       |         |   |        |

|  |    <--------------------------------+      |       | OEMI87 ---->| INTR87 |

|  |    |     | |                 |             |       |         |   |        |

|  |    |----------------------------> TSIR87   |       |         |   |   |    |

|  |    |     | |                 |      |      |       |   <---------|---+    |

|  |    <--------------------------------+      |       |         |   |        |

|  +----------+ |                 |             |       +---------+   +--------+

|       |       |                 |             |

|       |       |                 |             |

|     ENTGQQ    |                 |             |

|       |       |                 |             |

|       |       |                 |             |

|       V       |                 |             |

|  +----------+ |                 |             |

|  |  ENDX87  -----------------------> OEMS87   |

|  |    |     | |                 |      |      |

|  |    <--------------------------------+      |

|  |    |     | |                 |             |

|  |    |----------------------------> TSRR87   |

|  |    |     | |                 |      |      |

|  |    <--------------------------------+      |

|  +----------+ |                 |             |

|       |       |                 |             |

+---------------+                 +-------------+

|

V

Explaination of the flow of control.

------------------------------------

Microsoft provides  an  entry  point module ENTX6?.ASM ( e.g. ENTX6L.ASM

).  All Microsoft programs begin execution at a label in ENTR??   called

BEGXQQ.   Control  passes  from  BEGXQQ  to  the  routine  INIX87 via an

indirect  long  call.    INIX87   is   the   Microsoft   provided   8087

initialization routine.   INIX87  provides  for  all machine independent

8087 initialization.  INIX87 calls OEMR87.

Because of the numerous ways in which  interrupts  may  be  mediated  in

an 8086-8087  system  (  e.g.  8259 interrupt controller, NMI connection

) only the Oem can  provide  the  proper  interrupt  interface.   OEMR87

is  an  OEM  PROVIDED  ROUTINE  which  performs  ALL  MACHINE  DEPENDENT

INTIALIZATION OF AN 8087.

It IS THE RESPONSIBILITY  OF  OEMR87  to  install  an  8087  exceptional

condition handler.  This handler, called OEMI87, calls an 8087 exception

routine , INTR7, provided by Microsoft.  It  IS  THE  RESPONSIBILITY  OF

OEMI87 to  ensure  that  the  8086  DS  register is equal to that set by

ENTX6?.ASM and to call INTR7 if the 8087 is the source of the  interrupt

( see  the  generic  example  below  ).   The routine INTR7 may be found

in the MS library module COMR7.ASM.

Flow of control returns to INIX87 from OEMR87.   INIX87  then  calls  an

OEM PROVIDED  ROUTINE  called  TSIR87  which  tests  the installation of

the 8087 interrupt handler.  TSIR87 may be a dummy routine  if  no  8087

interrupt installation test is desired.

Flow of control returns to INIX87 from TSIR87.   Flow  of  control  then

returns to  BEGXQQ  from  INIX87.   Control  then passes to ENTGQQ - the

main program.  When ENTGQQ terminates, control passes to ENDX87.  ENDX87

is  a  Microsoft  supplied  routine  which  reverses  any  special  8087

preparations done by INIX87.  ENDX87 calls OEMS87.   OEMS87  is  an  OEM

PROVIDED ROUTINE  which  reverses  ANY  AND  ALL  MACHINE DEPENDENT 8087

preparations done by OEMR87.  Flow of control  returns  to  ENDX87  from

OEMS87.   ENDX87  then  calls  an  OEM  PROVIDED  ROUTINE called TSRR87.

TSRR87 tests that the 8087 interrupt handler INTR87  has  been  removed.

TSRR87 may  be  a  dummy  routine  if  test is desired.  Flow of control

passes from TSRR87 to ENDR87.  Flow of control then passes  from  ENDR87

to the ENTX??  module and the program terminates.

There is a global double word EXCX87, which is used to count the  number

of times  the  Microsoft  supplied  interrupt  routine INTR7 is entered.

The variable EXCX87 is cleared by INIX87 before  TSIR87  is  called  and

is incremented by one each time INTR7 is entered.

TSIR87  is  a  function  that  returns  a  non-zero value in the 8086 AX

register if the installation  of  INTR7  was  successful.   Since  INTR7

increments EXCX87,  a possible test for successful installation of INTR7

would be to execute a software interrupt to  OEMI87  that  causes  INTR7

to be  called.   Upon  return  from the interrupt EXCX87 could be copied

to AX.

It is THE RESPONSIBILITY OF THE OEM to  set  the  global  byte  variable

CHPRQQ in  OEMX87.   If  non  ZERO,  this byte indicates the presence of

an 8087 in the system.  Both the 8087  runtime  and  the  8087  emulator

check the  status  of  this  byte.   To insure that the MS IEEE emulator

will always be used simply set CHPRQQ to ZERO in OEMR7.ASM.

Rather than  being  called  directly,  INIX87  and  ENDX87 are called by

ENTX6?  indirectly through  a  special  segment  EINQQQ.   This  segment

allows all 8087 functions to be "decoupled" from ENTX6?.

EINQQQ  SEGMENT WORD COMMON 'DATA'

InitializeCodeAddress   DD  INIX87

ExitCodeAddress         DD  ENDX87

EINQQQ  ENDS

A generic example of an OEMR87

------------------------------

3.11 Release

------------

The  following  is  a  generic  example  of  the module OEMR87.  In most

instances, only those portions of  the  following  module  dealing  with

interrupt dispatching  need be changed ( e.g. issuing an EOI instruction

to an 8259 controller ).  The example shows how  a  user's  DS  register

may be restored when 8087 interrupts occur.

;-----------------------------------------------------------------------

;

; OEMR87 - Oem dependent 8087 hardware initialization.

;

; This module contains any hardware dependent  code  necessary  for  the

; operation of  the  i8087  with MS languages.  This module installs the

; i8087 interrupt handler INTR87.  The procedures  in  this  module  are

; called from  INIX87  and  ENDX87 which are in turn called from ENTX??.

; The two word declaration of EXCX87 is required.

;-----------------------------------------------------------------------

NAME  OEMR87

TITLE OEMR87 -   OEM 8087 Interrupt     Handler      Installation

;                for MS-Pascal,Fortran.

PAGE

;-----------------------------------------------------------------------

;       MicroSoft supplied  8087  OEM  independent  interrupt   dispatch

;       routine.

;

EXTRN   INTR87:FAR

;-----------------------------------------------------------------------

;       MicroSoft end of program entry point.

EXTRN   ENDXQQ:FAR

;-----------------------------------------------------------------------

;       Utilities.

WASTETIME MACRO

PUSH    AX

POP     AX

PUSH    AX

POP     AX

ENDM

SAVEREG MACRO   R

IRP  PR,<R>

PUSH    PR

ENDM

ENDM

RESTOREREG      MACRO   R

IRP  PR,<R>

POP     PR

ENDM

ENDM

;---------------------------------------------------------------------

;       Assembly constants.

;---------------------------------------------------------------------

include     SYSI.ASM

PENDINGBIT      EQU     80H     ; 8087 interrupt pending bit.

TWOPOINTZERO    EQU     0       ;

CONTROLLER_PORT EQU     0       ; Guess at OEM controller port number.

IF PCDOS

PORT_C       EQU     62H

ENDIF

IF PCDOS OR MSDOS OR SCP

i8087IntNum     EQU     02H

OPSYS        EQU     21H

SETVECOP     EQU     25H

GETVECOP     EQU     35H

CNTRLCOP     EQU     23H

VERSIONOP    EQU     30H

ENDIF

IF OTHERS

i8087IntNum  EQU     02H

ENDIF

;---------------------------------------------------------------------

DATA    SEGMENT PUBLIC  'DATA'

EXTRN   CHPRQQ:BYTE     ; this must be set to non 0 if 8087 is present and

; it is to be used for fixup on the fly

PUBLIC  EXCX87

EXCX87  DW      0                       ; 8087 exception counter.

DW      0

LABX87     DB      '<8087>'

EOIX87     DB      0

PRTX87     DB      0

SHRX87     DB      0

INTX87     DB      i8087intnum

INTOFFSET  DW      0

DATA    ENDS

OEMR87  SEGMENT  PUBLIC  'CODE'

DGROUP  GROUP   DATA

CGROUP  GROUP   OEMR87

ASSUME CS:OEMR87,DS:DGROUP,SS:DGROUP,ES:DGROUP

;---------------------------------------------------------------------

;       Storage for initial Pascal-Fortran Data segment.

;       8087 control word.

ODVRQQ          DW      0       ; Old vector.

DW      0

DATS87          DW      0

CTSR87          DW      0

EVEN

;---------------------------------------------------------------------

;       OEMI87 - OEM 8087 Interrupt procedure.

;

;       PROCEDURE OEMI87;

;

OEMI87  PROC    FAR

PUSH AX                 ; Save DS of interrupt context.

PUSH DS

MOV  AX,CS:DATS87       ; Restore DS of user program.

MOV  DS,AX

;

; Test for 8087 interrupt.

;

FNSTSW CS:CTSR87

WASTETIME

TEST   CS:CTSR87,PENDINGBIT

JZ     OVER8087         ; Not an 8087 interrupt.

; Forward interrupt to old vector routine.

SAVEREG         <BX,CX,ES,SI,DX,DI,BP>

;

; Must be an 8087 interrupt.

;

; Make user   data   segment   available

; to INTR87.

MOV  ES,AX

CALL INTR87             ; 8087 interrupt dispatch.

MOV  AX,CS:DATS87

MOV  DS,AX

;

; EOI -  End of interrupt command to interrupt dispatcher.

;

AND     EOIX87,0FFH     ; Is the system configured as EOI.

JZ      OVERI8259

MOV     AL,EOIX87       ; Load up EOI instruction.

XOR     DH,DH

MOV     DL,PRTX87

OUT     DX,AL           ; Reset 8259 interrupt controller.

OVERI8259:

RESTOREREG      <BP,DI,DX,SI,ES,CX,BX>

OVER8087:

INC   EXCX87                     ; Count exceptions via this vector.

JNO   O8087

INC   EXCX87+2

O8087:

TEST SHRX87,1

JZ   DONE8087

POP   DS

POP   AX

JMP   CS:(DWORD PTR ODVRQQ)      ; We should never return from here.

DONE8087:

STI

POP   DS

POP   AX

IRET

OEMI87 ENDP

;---------------------------------------------------------------------

;

;       OEMX87 - Perform OEM specific initialization of the 8087.

;

;       PROCEDURE OEMX87

;

PUBLIC  OEMX87

OEMX87  PROC   FAR

;

; Test for existence of 8087.

;

MOV     BYTE PTR CHPRQQ,1       ; Chip is present if this is non-0

FNINIT                          ; Initialize the 8087.

XOR     AX,AX                   ; Clean AX.

MOV     EXCX87,AX               ; Clear temporary.

FNSTCW  EXCX87                  ; Issue 8087 instruction with no wait.

WASTETIME                       ; Waste some time.

OR      EXCX87,AX               ; Did any status word come back?

IFE     FRONTENDS                       ; Always use emulator for front ends.

JNZ     GOT8087                 ; No 8087  present or 8087 malfunction.

ENDIF

MOV     BYTE PTR CHPRQQ,0

RET

GOT8087:

SAVEREG       <BP,DX,ES,SI,DI,BX,AX,CX,DS>

; Initialize 8087 exception counter.

;

XOR  AX,AX

MOV  EXCX87,AX

MOV  EXCX87+2,AX

; Save this executing programs DS.

;

MOV CS:DATS87,SS

MOV  AL,BYTE PTR INTX87         ;

IF SCP OR PCDOS OR MSDOS OR OTHERS

MOV  CS:(BYTE PTR INTINS+1),AL  ; Set up test interrupt number.

ENDIF

MOV  BYTE PTR INTOFFSET,AL      ; Set up interrupt offset.

MOV  CL,2                       ;

SAL  WORD PTR INTOFFSET,CL      ; *4

; Save old interrupt vector.

;

IF PCDOS OR ( MSDOS AND TWOPOINTZERO )

;

; Ask operating system for vector.

;

MOV     AL,BYTE PTR INTX87      ; Interrupt vector number.

MOV     AH,GETVECOP             ; Operating system call interrupt.

INT     OPSYS                   ; Call operating system.

MOV     CS:ODVRQQ,BX            ; Squirrel aways old vector.

MOV     CS:(ODVRQQ+2),ES

ELSE

;

; Do a read into the vector space.

;

PUSH    DS

XOR     BX,BX                   ; Clear BX.

MOV     AX,CS                   ; Set up DS:SI, ES:DI.

MOV     ES,AX                   ; ES <-- CS.

XOR     AH,AH                   ; Compute vector offset.

MOV     SI,INTOFFSET            ;

MOV     DS,BX                   ; DS <-- 0.

CLI                             ; Disable 8086 interrupts.

MOV     DI,OFFSET ODVRQQ        ;

CLD                             ; Forward.

MOVSW                           ; Move from   0:INTX87   to

MOVSW                           ; CS:ODVRQQ.

STI                             ; Restore 8086 interrupts.

POP     DS

ENDIF

IF MSDOS OR PCDOS

; Have operating system intall interrupt vectors.

;

MOV     DX,OFFSET OEMR87:OEMI87 ; Load DX with 8087 interrupt handler.

MOV     AH,SETVECOP             ; Set interrupt vector code in AH.

MOV     AL,BYTE PTR INTX87      ; Set vector number.

MOV     CX,CS                   ; Move current CS to DS for opsys calls.

MOV     DS,CX

INT     OPSYS                   ; Install vector.

MOV     DX,OFFSET OEMR87:CTCR87 ; Load DX with 8087 control C handler.

MOV     AH,SETVECOP

MOV     AL,CNTRLCOP

INT     OPSYS                   ; Install vector.

ELSE

MOV     BP,SP                   ;

MOV     DS,[BP]                 ; DS <-- Original DS.

MOV     ES,BX                   ; ES <-- 0

MOV     DI,INTOFFSET            ; Absolute vector address.

MOV     AX,OFFSET OEMR87:OEMI87 ; Interrupt routine       offset

; ( by    convention    relative

; to the   current  cs:assume  ).

CLI

STOSW

MOV     AX,CS                   ; Code segment.

STOSW

STI                             ; Restore 8086 interrupts.

ENDIF

RESTOREREG < DS,CX,AX,BX,DI,SI,ES,DX,BP>

RET

PUBLIC  TSIR87

;--------------------------------------------------------------------

;       TSIR87 - Test installation of 8087 interrupt vector routine.

;       Suggestion:

;           If testing is undesirable, simply move 1 to AX.

;

;       PROCEDURE TSIR87;

;

TSIR87  LABEL FAR

IF SCP OR PCDOS OR MSDOS OR OTHERS

INTINS: INT    i8087intnum

ENDIF

MOV    AX,DGROUP:EXCX87

RET

OEMX87  ENDP

;---------------------------------------------------------------------

;       OEMS87 - OEM specific image rundown, 8087 clean up.

;       Called as part of ENDXQQ sequence.

;

;       PROCEDURE OEMS87;

;

PUBLIC  OEMS87

OEMS87  PROC  FAR

; Restore old interrupt vector.

IF MSDOS OR PCDOS

SAVEREG         <ES,DS,DX,AX>

; Was any vector saved to reinstall?

;

LES  AX,CS:(DWORD PTR ODVRQQ)

MOV  DX,ES

OR   AX,DX

JZ   NOTINSTALLED

; Request operating system to load vector.

;

MOV  AH,SETVECOP

MOV  AL,BYTE PTR INTX87

LDS  DX,CS:(DWORD PTR ODVRQQ)

INT  OPSYS

RESTOREREG      <AX,DX,DS,ES>

ELSE

SAVEREG         <BX,CX,ES,DS,DI,SI>

LES  BX,CS:(DWORD PTR ODVRQQ)

MOV  CX,ES

OR   CX,BX

JZ   NOTINSTALLED

CLI                             ; Disable 8086 interrupts.

; Load vector manually.

;

XOR BX,BX                       ; Clear BX.

MOV ES,BX                       ; Segment offset.

MOV DI,INTOFFSET                ; Interrupt vector offset.

PUSH CS                         ;

POP  DS                         ;

MOV SI,OFFSET ODVRQQ            ;

CLD

MOVSW                           ;

MOVSW                           ;

STI                             ; Enable 8086 interrupts.

RESTOREREG      <SI,DI,DS,ES,CX,BX>

ENDIF

IF MSDOS OR PCDOS

STC                             ; Set carry bit for ^C interrupt exit.

ENDIF

NOTINSTALLED:

RET

;--------------------------------------------------------------------

;       CTCR87 - Control C^ exit point.

PUBLIC  CTCR87

CTCR87  LABEL    FAR

JMP      ENDXQQ  ; Jump back into ENTX6? and close files.

;

;--------------------------------------------------------------------

;       TSRR87 - Test removal of 8087 interrupt vector routine.

;       Suggestion:

;          If testing is undesirable, simply exit with a RET.

;

;       PROCEDURE TSRR87;

;

PUBLIC  TSRR87

TSRR87  LABEL FAR

RET

OEMS87  ENDP

OEMR87  ENDS

END

SYSI.ASM

;-----------------------------------------------------------------------

; System switches definition file.

;

; Various versions  of  this file (e.g. with names sysiu.asm, sysil.asm,

; etc.)  exist.  The file  contains  definitions  of  assembly  switches

; used  by  other  runtime  routines  to  insert  special-case  code for

; particular systems.

;

; Runtime routines that need to know which system they are  being  built

; for should  include  the  file  "sysi.asm".  During the build process,

; the  proper  version  of  sysi.asm   should   be   copied   into   the

; currently-connected directory and given the name "sysi.asm".

;-----------------------------------------------------------------------

;-----------------------------------------------------------------------

; TRUE and FALSE.

;-----------------------------------------------------------------------

TRUE  EQU 1

FALSE EQU 0

;-----------------------------------------------------------------------

; Default radix for assembler.

;-----------------------------------------------------------------------

.RADIX  10

;-----------------------------------------------------------------------

; Define Operating system switches.

;-----------------------------------------------------------------------

XENIXKERNAL     EQU     FALSE    ; use in XENIX kernel only

XENIXRUNTIME    EQU     FALSE    ; use in XENIX runtime only (sysiu.asm)

PCDOS           EQU     FALSE    ; use in PCDOS environments (sysis.asm)

MSDOS           EQU     FALSE    ; use in MS-DOS environments (sysil.asm)

SCP             EQU     FALSE    ; Seattle Computer Products equipment

OTHERS          EQU     FALSE    ; CP/M 86 machines (sysim.asm)

;------------------------------------------------------------------------

; Define initial emulator interrupt number;  uses 10 consecutive vectors.

;------------------------------------------------------------------------

IF XENIXRUNTIME OR XENIXKERNAL

BEGINT  EQU     240; XENIXKERNAL beginning interrupt number.

ENDIF

IF MSDOS OR PCDOS

BEGINT  EQU     52

ENDIF

IF OTHERS

BEGINT  EQU     52

ENDIF

;-----------------------------------------------------------------------

; Encode-Decode Assembly Booleans.

;-----------------------------------------------------------------------

FUDGE EQU FALSE;  Use 8087 BCD encode and constant instructions.

LOG87 EQU TRUE ;  Use long multiplication log10 calculations.

ROUNDTOINT EQU TRUE ;  Software or hardware supports 8087 round to integer.

ESTIMATE EQU FALSE;  Estimated log - a calculus approximation.

CHIPCONST EQU FALSE;  Use on chip 8087 constants.

TESTING  EQU FALSE;

SMALLTABLES EQU FALSE;  True if small but slow powers of ten function.

EMULATOR EQU TRUE ;  True if emulator being used.

DENORMALS EQU TRUE ;  True if denormals are supported.

;-----------------------------------------------------------------------

; Define debugging information flag.

;-----------------------------------------------------------------------

INSTRUMENTED EQU FALSE