WORDSTAR CONFIGURATION FOR OKIDATA MICROLINE 80/82A/83A/84/92/93

PRINTERS AND THE OSBORNE EXECUTIVE I

NOTE: This file comes from Stephen Hill in Cupertino, Calif.

As I do not have an Okidata printer, I cannot directly check

the instructions, but they should be useful to those of you

who do have Oki printers.

Jim Switz, FOG Executive Librarian

26 December 1983

This information for Okidata Microline-series printer

installation comes from a factory bulletin dated August 15, 1983.

If you have any questions or problems after consulting your

computer manuals, contact your dealer or

Technical Support Group

OKIDATA Corporation

111 Gaither Drive

Mt. Laurel, N. J. 08054

609-235-2600   (ask for Technical Support)

The configuration is accomplished first by running the SETUP

program (ref. pp. 75-81, vol. 1, Executive Guide).

Place the CP/M disk in drive A.

Place the Wordstar disk in drive B.

Type SETUP (CR) after the A> prompt.

Screen 1 appears: "Select source for configuration"

Select:  From Drive B

Screen 2: "Select the items you want to change"

Select:  Key definitions (CR)

Move curser to "Arrow Settings", then (CR)

Screen 3:  "Select standard arrow key definition"

Move curser to "Wordstar Standard", then (CR)

Press (ESC) key to return to Screen 2.

Press (ESC) key again.

Screen 2.  Move curser to "Printer Port", then (CR)

Configuration is:

Baud rate = 1200

Protocol  = no protocol

Device Selection = Other (AUXIN:/AUXOUT:)

Initialization String = none

Move curser to "Parallel Port", then (CR)

Configuration is:

Protocol = Centronics

Device Selection = Printer (LST:)

Initialization String = <ESC>1  (Note: press Escape

key, release it, press 1 key, release it, then press

(CTRL)(CR)

Move curser to:  "Exit to Save Menu", then (CR)

Save Setup to: Drive B

Important note:  All of the above SETUP configurations

should be saved on your CP/M and SuperCalc disks, EXCEPT, under

Parallel Port, the Initialization String = none.  The <ESC>1

string instructs the printer to print in correspondence mode.

A menu-driven program named INSTALL, included on the

Wordstar package, is the second program to run.  This program

allows you to tell Wordstar what kind of printer you are using

and how it operates.  (References:  vol. 2, pp. 176-189 Executive

Guide, and Nov. 1983 FOGHORN, p. 32).

Wordstar operates many of the same functions as the

Microline Printer, including boldface, double strike,

underscoring, and other word processing features.

Insert Wordstar INSTALL in Drive A.

Insert Wordstar disk in Drive B.

Boot the system.

Answer the questions upon prompt.

You will then get to the Installation Menu.

Select "C" Menu of Printers

Select either "standard printer" or "Okidata ML84A" (if

you have this model).

Communications Protocol = "A" none

Driver = "A" operating system primary list device.

Select "D" Custom Installation of Printer

A menu will appear.

Select "F" Boldfacing (^PB)

The patches are as follows:

80        82A/83A        84/92/93

--        -------        --------

03        no patch           03

Select "I" Ribbon Selection (^PY)

The patches are entered as hexadecimal.  Precede each

number with a comma.  Press (CR) after each number. Eg.

,03 (CR)

"Alternate Ribbon Selection - first ^PY"

This patch toggles between underlining and not

underlining.

80        82A/83A        84/92/93

--        -------        --------

no patch  no patch            02

1B

43

(CR)

"Stardard Ribbon Selection - second ^PY"

80        82A/83A        84/92/93

--        ------         --------

no patch  no patch            02

1B

44

(CR)

Select "J" Vertical Motion (^PT and ^PV)

"Start Vertical Motion - ^PT" (superscript)

80        82A/83A        84/92/93

--        ------         --------

no patch  no patch            02

1B

4A

(CR)

"Stop Vertical Motion - ^PV" (subscript)

80        82A/83A        84/92/93

--        ------         --------

no patch  no patch            02

1B

4C

(CR)

VMI minimum value = -1

VMI range value   = -2

Line Feed Function Code = (empty)

Reverse Line Feed Function Code = (empty)

Select "O" User Defined Function

"User function #1"   (^PQ)

80        82A/83A        84/92/93    Mode

--        ------         --------    ----

01        01             01        80       = 16.5 cpi

1D        1D             1C        82A/83A  = 16.5 cpi

(CR)      (CR)           (CR)      84/92/93 = 12 cpi

(CR)      (CR)           (CR)

"User function #2"   (^PW)

01        01             01        80       = 5 cpi

1F        1F             1F        82A/83A  = 5 cpi

(CR)      (CR)           (CR)      84/92/93 = 5 cpi

(CR)      (CR)           (CR)

"User function #3"   (^PE)

01        02             01        80       = 10 cpi

1E        1D             1D        82A/83A  = 8.3 cpi

(CR)      1F             (CR)      84/92/93 = 17 cpi

(CR)      (CR)           (CR)

"User function #4"   (^PR)

01        01             01        80       = 10 cpi

1E        1E             1E        82A/83A  = 10 cpi

0         0              0         84/92/93 = 10 cpi

(CR)      (CR)           (CR)

Select "Q" Character Pitch

"Select alternate pitch"   (^PA)

80        82A/83A        84/92/93

--        ------         -------

02        02             02        80       = 8 lpi

1B        1B             1B        82A/83A  = 8 lpi

38        38             31        84/92/93 = corresp.

(CR)      (CR)           (CR)

"Select standard pitch"   (^PN)

02        02             02        80       = 6 lpi

1B        1B             1B        82A/83A  = 6 lpi

36        36             30        84/92/93 = data

(CR)      (CR)           (CR)

Notes:  1.  If you ML-84 printer is not a Step 2 printer,

then the codes for 12 cpi and 17 cpi will be switched.

2.  Correspondence mode is not supported in 17 cpi on the

ML-92 and ML-93.

------------------------

Wordstar (R) is a registered trademark of MicroPro International

Corporation.########