This article describes a table driven assembler I recently constructed

to help with my  investigation  of  the  new  one  chip  microcomputers.   At

present, my interest is in the Intel 8748 computer and the related Intel 8741

Universal  Peripheral Interface, both with user programable EPROMs.  With the

continued proliferation of new microprocessors, especially low end processors

too small for resident utilities, I felt the best approach was a table driven

cross-assembler.  Since my main system is an 8085-based system running  CP/M,

I had a large choice of source languages for the assembler.  I uses ML-80 for

several   reasons,   including   availability,   efficiency   and  structured

programming features.  ML-80 is a structured  assembler/compiler  written  in

1975 to combine the advantages of assembler level machine control with a good

set of control structures to produce readable and efficient code.  I wrote it

as  a  table  driven  compiler to make it easier to modify for use with other

8-bit machines.  In particular, all components of an  operation  code  except

immediate  operands  are  are  determined  by table lookup alone.  The target

machines for the assembler listed  here  are  the  Intel  8748/8048/8035  and

8741/8041.   A short companion article gives an explanation of the syntax and

semantics of ML-80 as well as operational notes not widely published.

General features of the Assembler

The  assembler  provides  as  basic  features  for  all  versions line

parsing, symbol table management and expression evaluation.  The line  parser

defines  a source line as a series of fields separated by blanks and tabs and

terminated by CR/LF.  The first field is an optional label starting in column

one, not over 6 characters long, optionally followed by a colon.  The  second

field  is  an  operation  code  not over 5 characters.  The third field is an

optional list of operands separated by commas.  The fourth field  treats  the

rest  of  the  line  as  comments.  Note that for some operations some of the

optional fields may actually be required.  To allow  for  longer  labels  and

operations  requires  changing  the declarations for OP, LABELL, LABELTAB and

adjusting the loop counters in  PARSELINE  and  GETNAMEVAL.   The  expression

evaluator  recognizes  labels,  numbers  and '$' for the current value of the

location counter (i.e.  the address of the instruction  containing  $).   ...

Note  that  his routine had an error if the instruction is two bytes long, th

has  now  been  fixed  (Trevor  Marshall).   Numbers  may  be   in   decimal,

hexadecimal,  octal  or  binary,  denoted  by  a  suffix  of  D,  H,  O, or B

respectively.  A number with no suffix is decimal.  The  first  character  of

numbers  must  always  be  digits  to  distinguish  them  from  labels.   The

expression evaluator also recognizes the operation symbols '+', '-', '>'  and

'<' for plus, minus, low byte and high byte respectively.  When used as unary

operations, the first operand is zero.  Low byte is the less significant byte

of  the  following  value  and  high  byte is the more significant byte.  For

example, >01234H is 34H and <01234H is 12H.

Customizing the assembler

The behavior of the assembler is controlled by two main tables and two

case statements (one in each pass).  The first table in the  listing  is  the

operand  table from label RR1 through RR31.  This table is coded using macros

RENTRY, RERROR, RNULL and RGO.  The second table is  in  the  declaration  of

OTAB  immediately  after the operand table.  This table, constructed with the

macro OENTRY, describes the operation codes for the assembler.  Each entry in

this table gives an operation code, e.g.  'ORG',  the  base  value  for  this

operation,  e.g.   '0', and names a starting point in the operand table, e.g.

'RR30'.  Each operation is applied to a serial search of this  table  for  an

exact  match (both length and contents), the operation value is saved and the

operand pointer is used to start the search for operands.  The operand  table

is  serially  searched  from the starting point given by the operation table.

The operand table contains several kinds of entries, some of which  exist  to

make  the  table  shorter  by sharing portions.  Entries created by the macro

RENTRY result i.  comparison of the start of the operand field with the table

entry.  An RENTRY consists of a match  string,  e.g.   'P1,#',  an  operation

correction  value,  e.g.   '49'  meaning 49H, which is added to the opcode to

produce the correct value for this operand, and a type  code,  e.g.   '[IM]',

which  drives  the  case  clauses for operand format in the assembler.  These

types are generated by macros which expand to small numbers.  The types which

are currently defined are error, immediate byte follows, same page  branches,

full  memory  call/jump,  psuedo  operation and operand complete.  The actual

meaning of a type is defined by  the  case  statements  in  PASS1  and  PASS2

described below.

Entries  created  by  the macro RNULL are for operations which have no

operands.  An entry of type RNULL is identical to RENTRY with  a  null  match

string.   Since  the  search is for a match on the first few characters of an

operand, if one operand form is a prefix of another for  the  same  operation

(say  P  and  P3)  the  longer one should come first, otherwise both P and P3

would match with P.

Entries  created  by  the  macro  RGO  are  used to share parts of the

operand lists.  An RGO entry gives  an  opcode  correction  value  and  names

another  part of the operand table.  When the table lookup routines encounter

an RGO entry, the search switches to the named part of the table after adding

the correction value to the opcode.  This correction allows  for  differences

in  the  spacing of different opcodes.  For example, the table entry RR4 ends

with [RGO '60' '[RR3]'] because most of the possible operands for ADD are the

same as for XCH, saving 10 entries.  RR3 is  part  of  another  space  saving

technique.   RR3  is  an internal label of the declaration for RR1 (and RR2),

thus, if the search falls through the entry after RR2, the effect is the same

as [RGO '0' '[RR3]'] without the overhead.  RGO allows  several  variants  to

join since only one can fall through, and also allows other variants by using

opcode corrections.

The  last  type of entry is generated by the RERROR macro.  This macro

signals that the operand field does not contain a  valid  entry.   Note  that

every path through the operand table must end with RERROR or RNULL.

PASS1 case statement

This  assembler  is  a  traditional  two pass assembler which resolves

label values on the first pass and generates code on the second pass.   These

two  passes are embodied in the procedures PASS1 and PASS2.  In each of these

procedures, a case statement is driven by the results of  the  operand  table

search.   In  pass  one,  the  case alternatives do processing to resolve all

labels.  The basic function of pass one defines all  labels  as  the  current

value of the location counter (the location counter is the assembler's analog

of the processor's program counter).  The main function of the case statement

is  to increment the location counter by the length of each instruction.  The

second function is to  provide  special  processing  for  psuedo  operations.

Psuedo  operation  processing  is  driven  by  a  second case statement.  The

driving value for this case  statement  is  the  opcode  value  since  psuedo

operations  don't generate an operation in the usual sense.  The case for EQU

modifies the definition of its label to the value  of  the  expression.   The

case  for  ORG  resets  the  location counter to its expression.  The DS case

increments the location counter by the value of its expression.

PASS2 case statement

The main action in pass two is code generation by the different  cases

with  appropriate  location  counter  adjustment (normally done by EMIT1, the

subroutine which emits a byte of code), again with special  processing  in  a

second case statement for pseudo-operations.  The calls to MAGNITUDEERROR are

made  when  a two byte value is to be emitted in one byte but is not the sign

extension of the single byte.

8741/8748 Assembler notes

The language currently recognized by the assembler is the union of the

operations for the Intel 8741 and 8748,  and  the  appropriate  Intel  User's

Manuals  are  the  primary  reference  on  valid  formats  and operands.  The

operations are divided into four types for this assembler.

Type 1.  This type is for operations with a single  immediate  operand

data byte, for example ADD A,#5

Type  2.   This  type  is  for the short conditional jump instructions

which are limited to destinations on the same page (256 byte  block)  as  the

next instruction, for example DJNZ R1,LOOP.

Type  3.   This  type is for call and jump in which three address bits

are part of the opcode and the remainder of the address is in a second byte.

Type 4.  This is a dummy type for psuedo operations.  These are

ORG     expression      ;set location counter to expression

label   EQU     expression      ;equate label to expression

[lab]   DS      expression      ;reserve expression bytes

[lab]   DB      expression      ;define byte with value expression

PRINT   ON               ;list assembly to printer

PRINT   OFF              ;no listing (except errors)

Most of these  are  standard  pseudo  operations.   The  PRINT  pseudo

operation  immediately  affects  the listing, making it possible to list only

parts of an assembly.  The initial state of the assembler is PRINT ON.

Type 5.  This case is for simple opcodes with operands included within

a single byte, for example MOV A,@R1.

Miscellaneous considerations

As compiled here, the assembler is limited to 255 labels by the  table

size  declaration,  but  the search strategy is general enough for any length

table.  To simplify I/O, the assembler also  presently  uses  the  fact  that

there  is  only  1K of EPROM in an Intel 8748/8741, and first generates an in

memory array of the result and finally writes a binary file  of  the  result.

For  larger  memories, such as an 8080, EMIT1 needs to be revised to write to

an output file, in a format such as Intel hex.  Since I wrote this on my CP/M

system, the I/O used is specific to CP/M but has been carefully  encapsulated

in only a few subroutines.  Procedure GETFC returns sucessive characters from

the  input  file,  and  thus  is  responsible  for  reading  disk sectors and

dribbling the contents out one character at a  time.   Procedures  PRINTALINE

and PRMSG call CP/M BDOS to type characters on the console.  PRINTALINE lists

lines  of  formatted  assembly output, while PRMSG types error messages.  The

procedures ZTFCB, OPENR and OPENHEX also  perform  CP/M  specific  functions.

These  procedures all assume the existence of a CP/M initialized file control

block which CP/M has parsed the required file name from the command  invoking

the  assembler.   For  another  DOS,  it  may be necessary to do this parsing

separately, and change the format assumptions made by these routines.   ZTFCB

initializes  a  file  control  block  prior  to  opening.  OPENR opens a file

control block to read the source file, and is called  twice,  once  for  each

pass.   OPENHEX  creates  a  new  file  with  suffix  SCC (now .COM -- Trevor

Marshall) and writes the memory image buffer of the  assembled  code  out  in

binary format.  Outside of these routines, the assembler makes no assumptions

about  the  file  structures  available except that it is a sequence of bytes

which can be read twice.  Conversion to another DOS should  be  fairly  easy,

and conversion to a non-disk system should also be feasible.

Program availability

The  ML-80 compiler is part of the CP/M users group collection, and so

is available from them.  The PL/M source for  ML-80  is  available  from  the

Intel  Users'  Group.   As  an  alternative, for $10 I will provide a 8 inch,

single density IBM format CP/M diskette with ML-80 and  the  source  for  the

assembler in this article on it.

###########################################################