MODIFICATIO� T� FMT.CO� T� ALLO� FORMA� O� EXTERNA� ADD-O� RIGIDS

WRITTE� 02/27/8�

Author�			RUS� PENCIN

SYSO� DALLA� CONNECTION

(214� 783-768� (ringback)

Th� followin� modification� ca� b� installe� usin� DD� an� mus� be

installed in order to allow you to format the rigids that you have

added to the 820-II.

The SA1403A controller will control up to 4 drives in any configuration.

It selects the drives as per the following table:

Unit 0	=	00	(normaly the floppy)

Unit 1	=	20	(add-on rigid #2)

Unit 2	=	40	(add-on rigid #1)

Unit 3	=	60	(system rigid)

To modify the FMT.COM program, bring it into memory using DDT:

A>DDT FMT.COM

The sign-on should look like:

0600 0100 xx

-

Now modify the following locations using the S command:

-S2F3 60 XX		(where XX is the new unit select)

-S31A 60 XX

-S320 60 XX

-S326 60 XX

-^C

A>save 5 FMTX.COM	(where X is the new unit character)

At this point you will need an FMTX.COM for each of your external rigids

because I haven't been able to add any user interface to select the drive

at run-time. Better this than nothing though.

If you should find that the values at these locations are not 60H, do a

dump for 02F0 and locate the 60's that are offset by the same amounts as

the ones above. There is only one other 60H in the file and it is very

near the beginning and should not be changed.

The program RIGID1.COM sets the external rigid to be unit 1 (20H) and the

program RIGID2.COM sets one rigid to be unit 1 (20H), and the other to be

unit 2 (40H).

Good luck with these mods, hope they make your new rigid extentions more

usable and easy to work with. If you have any problems, leave a message

on my RCP/M describing the problem, and I will get back to you with fixes

or suggestions.

Russ Pencin

Dallas Connection RCP/M

(214) 783-7684 (ringback 24hrs. 300/1200 baud)

[eof]

##########################################################################################################