CPU280 System Software Information

==================================

Tilmann Reh, 06 June 1993

System Software Version 1.1

---------------------------

The  new system software release 1.1 for the CPU280 is offered  freely

for  every  CPU280 owner or user. It is delivered as is,  without  any

guarantees.  So everyone may have a look at the software and find  out

what  he  has to change for getting it running with  his  environment.

However,  I tried to make the new version as modular and  portable  as

possible,  and  I  am always interested in receiving  bug  reports  or

suggestions for later releases.

The  original system software delivered by me may be distributed  only

complete.  The version message at signon must not be changed.  Changed

versions  might  be further distributed, if it  is  stated  absolutely

clearly that it is not the original version. Everyone who changes  the

system  software code (not only disk parameters etc.) should  add  his

initials  and  his  internal version number to the  BIOS  signon  (for

example,  for the 5th change being made by me, the signon  could  read

"V1.1tr5").

You may reach me using the following addresses:

Tilmann Reh, In der Grossenbach 46, D-57072 Siegen

Phone +49 271 312599, Fax +49 271 317359

Internet: tilmann.reh@hrz.uni-siegen.dbp.de

Before July 1993, the above ZIP code must be replaced by "W-5900". The

safest  way  to  reach me is regular mail -  fax  number  or  internet

address may change.

This  document  describes major features of the  new  system  software

release, especially implementing a particular harddisk drive.

P.S.:  Some users sometimes had problems to format diskettes with  the

new system (although the Format Manager and it's BIOS interface wasn't

changed).  Since I still don't know the cause of those  problems,  and

didn't manage to reproduce them yet, I ask you to keep your eyes  open

and inform me as soon as you experience some difficulties.

The modular approach

--------------------

The  new  system  introduces the concept of a  really  modular  driver

structure  and all options being selectable by  conditional  assembly.

The  modular driver structure applies especially to the disk  drivers,

where  the only public label of each driver are the extended DPH's  of

the  logical  drives.  The system kernel  addresses  the  driver  only

through  the use of the drive table and these XDPH's, so  the  drivers

should  be  freely exchangable, and it should be easy  to  create  new

driver modules covering different hardware.

Since  all options are choosen by conditional assembly,  new  hardware

support  routines  might simply be added to the original  source  code

as  another option (please inform me about such expansions, so  I  can

integrate them into further system software releases). For example,  a

harddisk   driver  for  OMTI  controllers  (the  old   XT-type   8-bit

controllers) will surely become another option in the HARD.280  module

(the option file is already prepared for this).

The modular disk drivers also are already prepared for being used with

the  next release of the boot loader program. For this  purpose,  they

contain code which is needed only with the boot loader, code which  is

needed  only with the system BIOS, and code which is needed for  both.

This, too, is controlled via conditional assembly. There is an  option

named LOADER, which is set true during assembly of the driver  modules

for the loader program, and false during assembly of the system  BIOS.

Since the option file is included in every source code module, we must

have  different option files for the loader and the system  BIOS.  The

appropriate  file (SYSTEM.LIB resp. LOADER.LIB) is then copied to  the

file OPTIONS.LIB (which is included in every source module) every time

the system BIOS or the loader program are assembled.

This is to give some information about the concept, and the meaning of

the  new option files. Don't mess around with the LOADER flag! I  have

not  yet started programming the new loader program, so it  will  last

some  time  before it makes sense to ask me about.  However,  the  old

loader  version  (1.02) will boot the new system from both  EPROM  and

disk  without  any problems (even if the new system does not  use  all

information the loader is offering).

Harddisk installation

---------------------

One  of the great features of the system release 1.1 is  the  harddisk

driver which allows for easy installation of IDE harddisks  (connected

via my IDE interface board) by the use of macros. One needs to  change

only  few  equations to get the new system running with  a  particular

harddisk.

Every  harddisk is described by it's operating parameters,  which  are

the number of cylinders, heads and sectors. The so-called native  mode

of  IDE drives uses the physical parameters as  operating  parameters.

Some  drives  support pseudo-native modes which have  some  parameters

modified  by powers of two, for example twice the number of heads  and

half  the  cylinder count. Many drives allow for free setting  of  the

operating parameters and calculate the physical values internally,  as

long  as  the  emulation fits into the total number  of  sectors  (the

drive's capacity). These emulation modes (including pseudo-native) are

for  the  "compatibility"  of the IBM  "clowns",  where  the  original

harddisk controller WD 1010 could control only 1024 cylinders. So with

modern  harddisks which have far more cylinders, these are  mapped  to

logical drives with fewer cylinders.

Since I don't know for sure if modern IDE drives also mask the  higher

bits of the cylinder register, you should also use an emulation  which

has less than 1024 cylinders (logical). Pseudo-native modes are to  be

preferred, since they probably are slightly faster. So, the first step

is to fix the operating parameters of the drive.

The  next thing to do is the specification of the logical drives,  the

partitions.  One physical harddisk drive may be splitted into  several

logical  drives, so there are more separated areas to store files  in.

Remember,  with  CP/M-3 you have only 16 user areas for  each  logical

drive. So if you want to put all your files covering one theme into an

own  area, you will need many partitions if you have many  themes.  To

give  you an idea of the size of a partition: having files  of  normal

size  (text files, programs, libraries, everything mixed), with  about

100 files per drive/user area, this means about 1600 files per  drive,

representing  about 10 to 15 megabytes. If you are planning  to  store

large  files on one partition only, you might as well choose a  larger

partition  for this purpose. If you want to have fewer files in a  d/u

area, choose more smaller partitions (if possible).

The system software release 1.1 supports 9 harddisk partitions without

changes.  By removing the optional semiconductor pseudo-drives  and/or

unused  disk drives, you might get some more. Partition sizes  can  be

defined by the number of cylinders.

After  you have defined how many partitions of which size you want  to

install,  you must define the block size of each  partition.  Remember

that you can allocate a maximum of 16 blocks to the directory. This is

a very important limit! With 4k blocks, there are max. 2048  directory

entries, enough for about 10-15 MB of mixed files when datestamping is

used (highly recommended!). With greater blocks, you get more room for

the  directory, while needing less - this is great! However,  blocking

losses  rise as they are half the blocksize statistically (already  4k

per  file when using 8k blocks). So you should choose the  block  size

(and  the partition size) depending on the type of files you  want  to

store in each partition. Then, the number of directory entries must be

defined accordingly.

The  last  thing  to do is caused by the  16-bit  arithmetics  of  the

assembler. I use an equation within a macro to calculate the number of

blocks  in each partition. For partitions greater than or equal to  32

MB,  an intermediate value will overflow and result in a  wrong  block

count. So for every partition which is concerned, you have to do  this

calculation  manually  and  enter the block count  directly  into  the

source code.

These  are  the four things to do, with some information where  to  do

them:

1. Fix the drive's operating parameters (cylinders, heads, sectors).

The  operating parameters are defined in three equations  at  the

beginning of the HARD.280 module.

2. Define number and size of partitions.

The  number  of  partitions  is  contained  in  the  option  file

SYSTEM.LIB  (!).  The  size of each  partition  is  defined  with

equations at the beginning of HARD.280.

3. Define the block size and directory capacity of each partition.

The block size of each partition is defined immediately following

the partition size definition, at the beginning of HARD.280.  The

block  size is defined as number of sectors per  group  (=block),

where a sector is always 512 byte.

The  number  of directory entries and  the  according  allocation

bytes must be defined directly within the DPB's, at the very  end

of  HARD.280. Partitions which are equal in capacity, block  size

and directory size, may share the same DPB. Change the number  of

different  DPB's to your requirements by adding or  removing;  be

sure to declare all needed DPB labels. Take care that if you have

too  much  DPB's, the resident BIOS portion will  need  3  memory

pages instead of two, reducing your TPA size by 256 byte.

4. Check for 32-MB overflows and correct manually.

If there are partitions of 32 MB or greater, find the source code

line where the block count of each partition is calculated  (near

the  end  of HARD.280, within the IRPC macro). This  line  should

read:

Blocks&i aset Part&i*Heads*Secs/SpG&i

Add  a new line !after! this definition which contains the  right

value for the partition in question, for example:

BlocksK aset 5036

if your partition K is sized about 40 MB and uses 8k blocks. Take

care  to use an integer division which simply cuts the  remainder

(do not round!).

One thing which I didn't mention yet: there's a signon message in  the

harddisk  driver which is displayed after  successful  initialization.

This  message  usually contains the harddisk's capacity and  the  used

logical  drives (the letter from H: to anywhere). This message  should

of course be adapted to the used harddisk and partitioning data.

After having created the new system file using the SYS make-file,  the

GENCPM utility should be run manually in any case, explicitly checking

the buffer sizes and definitions. Boot the new system from floppy disk

first,  before burning it into EPROM's (BTW, the GENEPR utility  still

works with the new system).

The  new  system  should be able to address  the  harddisk  partitions

perfectly right from the start. If you experience any problems, please

inform me about them as detailed as possible.

After  having booted the new system first, the directory structure  of

all partitions must be created. For this, fill the complete  directory

area with E5 bytes (for example using DU), except for the 60h position

in  every  record which should be 21's (the latter can  also  be  done

using the CP/M-3 command INITDIR). After that, use the SET command  to

set  the  partition's directory label and activate the  date  stamping

(create & update recommended).

ZPM 3

-----

There is a replacement for the CP/M-3 BDOS, written completely in  Z80

code.  Thus,  it  is much more efficient and  also  offers  additional

functions  compared  to the original CP/M-3.  Unfortunately,  it  also

seems to contain some bugs and some mis-specifications (like  register

usage  restrictions for the BIOS), at least in the actual release  09.

Although I took care of the details I knew about ZPM3 when programming

the  new BIOS, it still does not run error-free. Since the contact  to

the  (australian)  author of ZPM3, Simeon Cran, has broken,  we  don't

seem  to  have  the support needed to get it running.  If  we  someday

receive  a  release of ZPM3 without those bugs, this BIOS  should  run

with it without being changed.

- End of this document -

######################